# Formal Method Specification — Root Cause Analysis (RCA) using Knowledge Graphs

Version 3.0 | Matrix Profile · MMD · TSKR Integration

## 0. Assumptions

The workflow as specified assumes three pre-existing infrastructure components are already populated before the RCA pipeline runs:

- Neo4j KG — already built from MBSE models (system/component hierarchy) and pre-populated with asset nodes, failure mode nodes from FMEA, sensor nodes, and historical event nodes from prior anomalies and maintenance records.

  - Set of toml schemas defined

- Chroma vector store(s) — already ingested and chunked documents (CRs, WOs, SOPs, ECAs) with embeddings, one collection per document type as originally designed.

- Telemetry historian — accessible via API, providing raw time series on demand for the analysis window.

## 1. Purpose & Scope

Purpose: Define a formal, machine-readable, auditable method to perform RCA for nuclear power plant equipment using a knowledge graph (KG) built from MBSE and engineering repositories, telemetry, and maintenance/governance systems.

Scope: Single-event RCA augmented by historical events and corpora. Two operational modes are supported:

- Reactive: engineer faces an anomaly now — assist fast diagnosis.

- Proactive: surface degradation patterns before failures occur (precursor library, Section 7.4).

The method covers Steps 1–10: event framing → telemetry snapshot → KG narrowing → KG-based causality → evidence retrieval → Ishikawa assessment → effectiveness & compliance review → LLM synthesis → analyst review → write-back.

Assumptions:

- KG (Neo4j) and vector store (Chroma or equivalent) are available.

- Telemetry store (historian/Spark) and CMMS/PLM/EAM are accessible via APIs.

- Users are system engineers with authority to make final decisions; LLM outputs are advisory only.

- All Steps 1–7 are fully deterministic and seeded where stochastic methods are used, ensuring reproducibility of causality_candidates.json for identical inputs.

## 2. Key Definitions (Formal)

Foundational distinction — Equipment vs. Component:

- Equipment (Asset): A physical, uniquely identified plant instance (e.g., Pump P-101A). Events occur on assets.

- Component: An engineering element from the MBSE architecture (e.g., rolling-element bearing). Failure modes live on components. Assets realize components via the REALIZES KG edge.

Event (E):

E = {event_id, timestamp_start, timestamp_end, asset_id, component_id?,

severity, event_type, trigger_source, temporal_type,

symptom_signature, analysis_window}

event_type    ∈ {SYMPTOM, DEGRADATION, INTERVENTION, FAILURE}

trigger_source ∈ {alarm, operator_observation, surveillance_test,

predictive_monitoring, scheduled_inspection, other}

temporal_type  ∈ {interval_native, point_native, synthetic_interval}

Analysis Window (W):

W = {start, end} where:

start = event.timestamp_start − Δt_pre

end   = event.timestamp_end   + Δt_post

Δt is severity-dependent (Section 8.1).

Subgraph (S):

S = {subgraph_id, nodes[], edges[], hop_limit, kg_snapshot_version, generated_at}

Failure Mode (FM):

FM = {fm_id, name, superclasses[], mechanism[], detection_methods[],

expected_symptoms[], expected_anomaly_pattern,

expected_latency_min_days, expected_latency_max_days, rpn}

Anomaly (An): Output of matrix profile applied to a univariate signal.

An = (anom_detect, t_in, t_end)                        # detection only

An = (diagnosis,   t_in, t_end, source_fm_id)          # with diagnosis

An = (prognosis,   t_est[], RUL[])                     # with prognostics

Anomalies are interval-typed by definition (t_in, t_end). They are the primary input to TSKR temporal mining.

MMD Correlation Score: Statistical measure (Maximum Mean Discrepancy) of whether a time series distribution differs significantly before vs. during/after an event. Produces a p-value and effect size per (event, signal) pair. Used to validate that a detected anomaly corresponds to a real distributional shift.

TSKR Pattern: A temporal relationship label assigned to an event-type pair (A, B) derived from mining the historical event corpus:

TSKR_label ∈ {simultaneous, precedes(Δt_mean, Δt_std, n_cases), unordered}

- simultaneous: A and B co-occur within epsilon tolerance — likely co-symptoms of a shared upstream cause, not a propagation chain.

- precedes(Δt_mean, Δt_std, n_cases): A consistently precedes B with measurable lag across n historical cases — directional causal candidate.

- unordered: no consistent pattern found across available history.

KG Path Strength: Numerical score [0,1]. See Section 5.3.

Confidence Object: Five-dimensional structure. See Section 6.

RCA Card: Structured result containing hypotheses, evidence, actions, provenance. See Section 4.6 and spec/schemas/rca_card.json.

## 3. Overall Architecture

┌──────────────────────────────────────────────────────────────────┐

│                        ORCHESTRATOR                              │

│  Enforces data flow · Validates schemas · Manages checkpoints    │

└───────────────────────────┬──────────────────────────────────────┘

│

┌────────────────────▼─────────────────────┐

│        Step 1: Event Framing             │

│  Output: event.json                      │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│     Step 2: Telemetry Snapshot           │

│  Matrix Profile → anomaly intervals      │

│  MMD → event×signal correlation          │

│  Output: telemetry_summary.json          │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│       Step 3: KG Narrowing               │

│  Output: kg_context.json                 │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│  Step 3A: TSKR Temporal Pattern Mining   │

│  Input: kg_context.past_events +         │

│         validated anomaly event stream   │

│  Output: tskr_patterns.json              │

│  Annotates KG edges with temporal labels │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│  Step 4: Causality Candidate Generation  │

│  Input: kg_context + tskr_patterns +     │

│         telemetry_summary                │

│  Output: causality_candidates.json       │

└───────────────┬──────────────────────────┘

│

══════════╪═══════════════

HUMAN CHECKPOINT GATE 1

══════════╪═══════════════

│

┌───────────────▼──────────────────────────┐

│      Step 5: Evidence Retrieval          │

│  Output: evidence_bundle.json            │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│      Step 6: Ishikawa Evaluation         │

│  Output: ishikawa_matrix.json            │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│  Step 7: Effectiveness & Compliance      │

│  Output: pm_compliance.json              │

└────────────────────┬─────────────────────┘

│

┌────────────────────▼─────────────────────┐

│       Step 8: LLM Synthesis              │

│  Output: rca_card.json + rca_card.md     │

└────────────────────┬─────────────────────┘

│

══════════════╪══════════════════════

HUMAN CHECKPOINT GATE 2

══════════════╪══════════════════════

│

┌────────────────────▼─────────────────────┐

│    Steps 9–10: Review & Write-back       │

└──────────────────────────────────────────┘

### 3.1 Analytical Method Roles (Summary)

| Method | Input | Output | Role in RCA |
| --- | --- | --- | --- |
| Matrix Profile | Univariate time series | Anomaly intervals (t_in, t_end), motifs, discords | Anomaly detection; converts continuous signals into typed interval events |
| MMD | Time series + event timestamps | Per-(event, signal) correlation score + p-value | Validates that a detected event corresponds to a real distributional shift; filters false-positive anomalies |
| TSKR | Historical stream of validated typed events | Temporal labels on event-type pairs (simultaneous / precedes / unordered) | Establishes directional causal ordering between event types across historical cases; distinguishes coincidence from propagation |

These three methods are sequential and complementary. Matrix profile produces events; MMD validates them; TSKR mines their ordering. None of the three is redundant with the others.

### 3.2 Orchestrator

Responsibilities: enforce data flow order; schema-validate each artifact before passing downstream; manage two human checkpoint gates; graceful degradation on null outputs (explicit no_findings flag, never silent null propagation).

### 3.3 External Interfaces

| Endpoint | Method | Purpose |
| --- | --- | --- |
| /telemetry | GET | ?asset_id=&from=&to= |
| /ops/logs | GET | ?asset_id=&from=&to= |
| /kg/query | POST | Neo4j Cypher wrapper |
| /chroma/query | POST | Vector search |
| /kg/commit | POST | Write edges/nodes post-acceptance |
| /rca_card | POST | Store final accepted RCA card |

## 4. Data Models & JSON Schemas

Full machine-valid JSON Schema files live in spec/schemas/*.json. Examples below are illustrative. Schema files are authoritative in case of conflict.

### 4.1 Event — event.json

Key fields: trigger_source, temporal_type (interval_native / point_native / synthetic_interval), symptom_signature, analysis_window, related_cr_ids.

{

"event_id": "E2026-01-01-001",

"timestamp_start": "2026-01-01T03:14:00Z",

"timestamp_end":   "2026-01-01T03:45:00Z",

"asset_id":  "P-101A",

"severity":  "HIGH",

"event_type": "DEGRADATION",

"trigger_source": "alarm",

"temporal_type": "interval_native",

"symptom_signature": {

"description": "Sustained bearing temperature rise with high vibration on shaft X-axis",

"symptom_types": ["temperature", "vibration"],

"anomaly_pattern": "gradual_drift",

"affected_parameters": [

{"parameter": "bearing_temp", "sensor_id": "S-T-101",

"observed_value": 92.3, "unit": "degC",

"normal_range": {"min": 40, "max": 75}}

]

},

"analysis_window": {

"start": "2025-12-29T03:14:00Z",

"end":   "2026-01-03T03:45:00Z"

},

"related_cr_ids": ["CR-2026-0042"],

"rca_required": true

}

### 4.2 Telemetry Summary — telemetry_summary.json

Key additions over v2: mp_results block per signal (matrix profile outputs including discord index, motif index, anomaly intervals); mmd_results block per (event, signal) pair (MMD test statistic, p-value, effect size, validation decision); validated_anomalies carrying only MMD-confirmed anomalies forward; promoted_to_kg_event gate with explicit rationale.

{

"asset_id": "P-101A",

"event_id": "E2026-01-01-001",

"generated_at": "2026-01-01T04:00:00Z",

"window": {

"start": "2025-12-29T03:14:00Z", "end": "2026-01-03T03:45:00Z",

"baseline_start": "2025-11-01T00:00:00Z", "baseline_end": "2025-11-30T00:00:00Z"

},

"analysis_methods": ["matrix_profile", "mmd", "threshold_check"],

"signals": [

{

"sensor_id": "S-T-101",

"parameter": "bearing_temp",

"unit": "degC",

"stats": {

"mean": 84.1, "std": 4.2, "min": 71.0, "max": 92.3,

"p95": 91.8, "sample_count": 8640

},

"baseline_comparison": {

"baseline_mean": 61.8, "baseline_std": 2.1,

"delta_mean": 22.3, "deviation_sigma": 10.6

},

"mp_results": {

"window_size": 288,

"discord_index": 4312,

"discord_score": 0.83,

"anomaly_pattern": "gradual_drift",

"anomaly_intervals": [

{

"anomaly_id": "ANO-MP-001",

"t_in":  "2025-12-27T08:00:00Z",

"t_end": "2026-01-01T03:14:00Z",

"severity_score": 0.83,

"detection_method": "matrix_profile"

}

],

"motif_intervals": [

{

"motif_id": "MOT-001",

"t_in":  "2025-11-10T06:00:00Z",

"t_end": "2025-11-12T06:00:00Z",

"note": "Recurring 48h temperature elevation pattern — potential precursor motif."

}

]

},

"mmd_results": {

"event_id": "E2026-01-01-001",

"sensor_id": "S-T-101",

"l_front_hours": 72,

"l_dur_hours":   31,

"l_rear_hours":  48,

"mmd_statistic": 0.312,

"p_value": 0.003,

"effect_size": "large",

"validated": true,

"validation_note": "p < 0.01; distribution during anomaly interval significantly different from pre-event baseline."

},

"validated_anomalies": [

{

"anomaly_id": "ANO-MP-001",

"source_method": "matrix_profile",

"mmd_validated": true,

"pattern": "gradual_drift",

"t_in": "2025-12-27T08:00:00Z",

"t_end": "2026-01-01T03:14:00Z",

"severity_score": 0.83,

"promoted_to_kg_event": true,

"kg_event_id": "KGE-2026-0001",

"promotion_rationale": "severity_score >= 0.75 AND mmd_validated = true"

}

],

"within_normal_limits": false,

"data_quality": {

"missing_fraction": 0.001,

"flatline_detected": false,

"outlier_fraction": 0.003

}

}

],

"overall_assessment": {

"any_anomaly_detected": true,

"anomaly_count": 1,

"mmd_validated_count": 1,

"most_anomalous_signal": "S-T-101",

"earliest_anomaly_time": "2025-12-27T08:00:00Z",

"pattern_summary": "Gradual drift in bearing temperature from 2025-12-27, MMD-confirmed (p=0.003). Consistent with slow thermal degradation. No step-change or oscillation patterns."

}

}

Anomaly promotion rule (deterministic): An anomaly is promoted to a KG abnormal_event node if and only if:

- mp_results.discord_score >= config.mp_promotion_threshold (default 0.75), AND

- mmd_results.validated = true (p-value < config.mmd_alpha, default 0.05).

MMD validation is mandatory before KG promotion. A matrix-profile-detected anomaly that fails MMD validation is logged but not promoted — it is a candidate false positive.

### 4.3 TSKR Patterns — tskr_patterns.json

New artifact (Step 3A output). Records temporal relationship labels for all event-type pairs found in the KG subgraph neighborhood, mined from the historical event corpus. Annotates KG edges.

{

"subgraph_id": "SG-E2026-01-01-001",

"event_id":    "E2026-01-01-001",

"generated_at": "2026-01-01T04:08:00Z",

"min_cases_threshold": 5,

"epsilon_registry": {

"interval_interval": 3600,

"interval_point": 86400,

"point_point": 3600,

"note": "Epsilon in seconds per pair type. Derived from max(resolution_A, resolution_B) × epsilon_multiplier(2.0)."

},

"patterns": [

{

"pattern_id": "TSKR-001",

"event_type_a": "VIBRATION_HIGH",

"event_type_b": "TEMP_ELEVATED",

"temporal_type_a": "interval_native",

"temporal_type_b": "interval_native",

"operator_family": "interval_interval",

"tskr_label": "simultaneous",

"n_cases": 9,

"epsilon_used_seconds": 3600,

"interpretation": "Co-symptoms of shared upstream cause. Do not generate separate causal candidates — collapse into compound symptom.",

"kg_edge_annotation": {

"from_node": "VIBRATION_HIGH",

"to_node":   "TEMP_ELEVATED",

"edge_type": "TSKR_SIMULTANEOUS"

}

},

{

"pattern_id": "TSKR-002",

"event_type_a": "LUBE_DEGRADATION",

"event_type_b": "BEARING_TEMP_ANOMALY",

"temporal_type_a": "interval_native",

"temporal_type_b": "interval_native",

"operator_family": "interval_interval",

"tskr_label": "precedes",

"delta_t_mean_hours": 108,

"delta_t_std_hours":  36,

"n_cases": 7,

"epsilon_used_seconds": 3600,

"interpretation": "LUBE_DEGRADATION consistently precedes BEARING_TEMP_ANOMALY by ~4.5 days. Directional causal candidate.",

"kg_edge_annotation": {

"from_node": "LUBE_DEGRADATION",

"to_node":   "BEARING_TEMP_ANOMALY",

"edge_type": "TSKR_PRECEDES",

"properties": {

"delta_t_mean_hours": 108,

"delta_t_std_hours":  36,

"n_cases": 7

}

}

},

{

"pattern_id": "TSKR-003",

"event_type_a": "ALARM_TRIGGER",

"event_type_b": "WO_CREATION",

"temporal_type_a": "point_native",

"temporal_type_b": "point_native",

"operator_family": "point_point",

"tskr_label": "precedes",

"delta_t_mean_hours": 4.2,

"delta_t_std_hours":  2.1,

"n_cases": 14,

"interpretation": "Alarm consistently precedes WO creation by ~4h. Expected reactive pattern — not a causal signal, confirms detection-to-response latency.",

"kg_edge_annotation": {

"from_node": "ALARM_TRIGGER",

"to_node":   "WO_CREATION",

"edge_type": "TSKR_PRECEDES",

"properties": {"delta_t_mean_hours": 4.2, "delta_t_std_hours": 2.1, "n_cases": 14}

}

}

],

"insufficient_history_pairs": [

{

"event_type_a": "SEAL_LEAK",

"event_type_b": "BEARING_TEMP_ANOMALY",

"n_cases_available": 2,

"note": "Below min_cases_threshold (5). No TSKR label assigned. Temporal scoring falls back to FMEA latency window check only."

}

]

}

### 4.4 KG Context — kg_context.json

Unchanged from v2 except: tskr_edge_annotations field added to upstream_paths and downstream_paths path objects, carrying the TSKR label assigned to each edge after Step 3A mining. This transforms static MBSE topology edges into empirically annotated causal edges.

### 4.5 Causality Candidates — causality_candidates.json

Key addition: temporal_evidence block per candidate, explicitly recording which TSKR pattern supports the temporal score, what operator family was used, and whether the pair had sufficient history for TSKR or fell back to FMEA latency check.

{

"event_id": "E2026-01-01-001",

"subgraph_id": "SG-E2026-01-01-001",

"tskr_patterns_ref": "tskr_patterns.json",

"generated_at": "2026-01-01T04:15:00Z",

"scoring_config": {

"method": "weighted_sum",

"dimension_weights": {

"structural": 0.35, "evidence": 0.25,

"temporal":   0.20, "telemetry": 0.12, "governance": 0.08

},

"temporal_score_composition": {

"tskr_weight":         0.70,

"fmea_latency_weight": 0.30,

"note": "When TSKR pattern has n_cases >= min_cases_threshold, tskr_weight applies. When insufficient history, fmea_latency_weight = 1.0 (full fallback)."

},

"min_evidence_threshold": {

"min_dimensions_above_zero": 2,

"min_composite_score": 0.35

}

},

"candidates": [

{

"candidate_id": "CAND-001",

"rank": 1,

"hypothesis_type": "failure_mode",

"cause_label": "Bearing Wear due to Lubrication Loss",

"cause_fm_id": "FM-BEAR-WEAR-001",

"composite_score": 0.81,

"meets_evidence_threshold": true,

"scores": {

"structural": 0.88,

"evidence":   0.72,

"temporal":   0.87,

"telemetry":  0.83,

"governance": 0.40

},

"temporal_evidence": {

"tskr_pattern_id": "TSKR-002",

"tskr_label": "precedes",

"tskr_score": 0.91,

"tskr_n_cases": 7,

"tskr_delta_t_mean_hours": 108,

"tskr_delta_t_std_hours": 36,

"fmea_latency_score": 0.85,

"temporal_score_final": 0.87,

"fallback_used": false,

"operator_family": "interval_interval",

"interpretation": "LUBE_DEGRADATION precedes BEARING_TEMP_ANOMALY by ~4.5 days across 7 historical cases. Current onset at ~5 days — within 1σ of expected lag. Strong temporal support."

},

"score_rationale": {

"structural": "Direct 1-hop APPLIES_TO path; path_strength 0.88.",

"evidence":   "WO-2025-4821 confirms prior bearing service. CR-2026-0042 notes lubricant discoloration.",

"temporal":   "TSKR PRECEDES pattern (n=7, Δt 108±36h). Current lag ~120h — within 1σ. FMEA window 2–14 days also satisfied.",

"telemetry":  "Matrix profile gradual_drift on S-T-101, severity 0.83, MMD-validated (p=0.003). Matches FM expected pattern exactly.",

"governance": "PM lubrication check overdue by 17 days."

},

"coincidence_check": {

"vibration_temperature_collapsed": true,

"note": "VIBRATION_HIGH and TEMP_ELEVATED flagged as TSKR_SIMULTANEOUS (TSKR-001). Treated as compound symptom of same upstream cause, not separate candidates."

}

}

]

}

### 4.6 RCA Card — rca_card.json

Unchanged from v2 except: primary_hypothesis.citations now includes a tskr_pattern source type, enabling direct citation of a TSKR pattern (n_cases, Δt statistics) as evidence in the RCA narrative.

{

"primary_hypothesis": {

"citations": [

{

"claim_summary": "Lubrication degradation consistently precedes bearing temperature anomaly by ~4.5 days",

"source_type": "tskr_pattern",

"source_id": "TSKR-002",

"excerpt": "PRECEDES pattern, n=7 historical cases, Δt_mean=108h, Δt_std=36h. Current lag ~120h (within 1σ)."

}

]

}

}

## 5. Knowledge Graph (Neo4j) Design

### 5.1 Node Types

| Node Label | Key Properties |
| --- | --- |
| :Event | event_id, type, timestamp_start, timestamp_end, severity, asset_id, fm_id, temporal_type |
| :Asset | asset_id, location_id, system, train_id |
| :Component | component_id, model_id, location_id |
| :Sensor | sensor_id, type, calibration_date |
| :FailureMode | fm_id, name, superclasses[], mechanism[], expected_latency_min_days, expected_latency_max_days, rpn |
| :Procedure | procedure_id, revision, authority_level, text_hash |
| :PMPlan | plan_id, schedule_interval_days, asset_id |
| :ECR | ecr_id, status, implemented_date |
| :RCA_Card | rca_id, event_id, card_version, summary, analyst_id, accepted_at |
| :DataSource | source_id, native_type, timestamp_resolution_seconds, promotion_valid, promotion_method |
| :PrecursorPattern | pattern_id, fm_id, tskr_chain[], delta_t_mean_hours, delta_t_std_hours, n_cases, last_updated |

:DataSource node is the event source registry. Each data source (DCS historian, CMMS, alarm log, inspection system) is a node recording its native temporal type and resolution. Event nodes inherit temporal_type from their source at ingestion. This type tag drives operator selection in TSKR mining.

:PrecursorPattern node is the precursor library — a derived layer on the KG built from TSKR mining results. Each node encodes the empirically observed precursor chain for a given failure mode, enabling positional context in a degradation timeline.

### 5.2 Edge Types & Properties

| Edge | Direction | Key Properties |
| --- | --- | --- |
| REALIZES | (:Asset)→(:Component) | — |
| HAS_SENSOR | (:Component)→(:Sensor) | — |
| UPSTREAM_OF | (:Component)→(:Component) | path_strength |
| DOWNSTREAM_OF | (:Component)→(:Component) | path_strength |
| CONNECTED_TO | (:Component)↔(:Component) | path_strength, install_date |
| OCCURRED_ON | (:Event)→(:Asset) | — |
| AFFECTS | (:Event)→(:Component) | — |
| APPLIES_TO | (:FailureMode)→(:Component) | — |
| CONFIRMED_CAUSE | (:Event)→(:FailureMode) | confidence_obj, evidence_refs[], analyst_id, committed_at |
| MAY_CAUSE | (:Event)→(:Event) | confidence_obj, lag_min_days, lag_max_days, provenance |
| MITIGATED_BY | (:Event)→(:RCA_Card) | — |
| GOVERNS | (:Procedure)→(:Component) | authority_level |
| SCHEDULED_FOR | (:PMPlan)→(:Asset) | interval_days, last_executed, next_due |
| ANALYZES | (:RCA_Card)→(:Event) | — |
| PRODUCED_BY | (:RCA_Card)→(:Analyst) | accepted_at |
| FROM_SOURCE | (:Event)→(:DataSource) | — |
| TSKR_PRECEDES | (:Event)→(:Event) | delta_t_mean_hours, delta_t_std_hours, n_cases, operator_family |
| TSKR_SIMULTANEOUS | (:Event)↔(:Event) | n_cases, epsilon_seconds, operator_family |
| HAS_PRECURSOR | (:FailureMode)→(:PrecursorPattern) | — |

MAY_CAUSE → CONFIRMED_CAUSE promotion: Requires analyst decision = ACCEPTED at Gate 2, composite_score >= config.confirmed_cause_threshold (default 0.60), and confidence_label ∈ {medium, high, very_high}.

### 5.3 Edge Weighting & Path Strength

Default edge weights (config/kg_weights.yaml):

| Edge Type | Default Weight |
| --- | --- |
| APPLIES_TO | 0.95 |
| UPSTREAM_OF | 0.90 |
| DOWNSTREAM_OF | 0.85 |
| REALIZES | 1.00 |
| TSKR_PRECEDES | 0.92 — boosted above UPSTREAM_OF; empirical ordering is strong causal evidence |
| GOVERNS | 0.70 |
| CONNECTED_TO | 0.65 |
| TSKR_SIMULTANEOUS | 0.20 — low weight; simultaneous events are co-symptoms, not causal links |

Path strength formula:

path_strength(P) = (∏ weight(eᵢ)) × decay_factor^(N−1)

decay_factor = 0.85 (default, configurable)

Early pruning: discard any path containing an edge with weight < config.min_edge_weight (default 0.30).

## 6. Confidence Model (Formal)

### 6.1 Dimensions

| Dimension | Definition | Range |
| --- | --- | --- |
| structural | path_strength of best KG path from cause to event | [0,1] |
| evidence | authority-weighted document support from evidence_bundle | [0,1] |
| temporal | composite of TSKR partial-order score + FMEA latency check (see Section 7.3) | [0,1] |
| telemetry | match between matrix-profile anomaly pattern and FM expected_anomaly_pattern, weighted by MMD effect size | [0,1] |
| governance | PM/LCM/ECR compliance rate from pm_compliance.json | [0,1] |
| overall | discrete label from rule-set | enum |

overall ∈ {VERY_LOW, LOW, MEDIUM, HIGH, VERY_HIGH}

telemetry score computation:

pattern_match_score = 1.0 if mp_pattern == fm.expected_anomaly_pattern else 0.4

mmd_weight = map(effect_size): large=1.0, medium=0.7, small=0.4, none=0.1

telemetry_score = pattern_match_score × mmd_weight × (mp_discord_score)

This ensures that a correctly patterned anomaly with strong MMD validation scores higher than a pattern-matched anomaly with weak distributional support.

### 6.2 Aggregation Rules

# Veto rules (first match wins)

if structural < 0.20:                                    overall = VERY_LOW

if evidence < 0.20 AND structural < 0.50:               overall = LOW

# VERY_HIGH guard — requires strength across all dimensions

if structural >= 0.70 AND evidence >= 0.70

AND temporal >= 0.70 AND telemetry >= 0.60

AND governance >= 0.60:                              overall = VERY_HIGH

# Weighted sum

overall_score = (0.35 × structural) + (0.25 × evidence)

+ (0.20 × temporal)   + (0.12 × telemetry)

+ (0.08 × governance)

overall = map(overall_score):

[0.00, 0.20] → VERY_LOW

(0.20, 0.40] → LOW

(0.40, 0.65] → MEDIUM

(0.65, 0.85] → HIGH

(0.85, 1.00] → VERY_HIGH

Presets in config/confidence_rules.yaml: conservative (raises veto thresholds), permissive (lowers them).

## 7. Temporal Reasoning — Formalism

### 7.1 Event Temporal Typing

Each event node carries a temporal_type inherited from its :DataSource node:

| Source type | Native temporal type | Example |
| --- | --- | --- |
| DCS / SCADA historian | interval_native | Operating state duration |
| Condition monitoring (matrix profile output) | interval_native | Anomaly (t_in, t_end) |
| CMMS WO (open + close timestamp) | interval_native | Work order duration |
| Alarm log | point_native | Single trigger timestamp |
| Inspection finding | point_native | Date of observation |
| Failure notification / CR | point_native | Report date |
| WO with planned hours only | synthetic_interval | Promoted via planned duration |

Synthetic intervals must be flagged temporal_type = synthetic_interval for auditability. Operator selection reads type tags automatically.

### 7.2 Mixed Event Type Operator Selection

The valid temporal operator family depends on the native types of both events. Applying operators inconsistently across mixed types produces invalid patterns.

| Pair type | Valid operator family | Config reference |
| --- | --- | --- |
| Interval × Interval | Full TSKR set: coincidence, partial order, meets, overlaps, starts, during | temporal_operators.yaml §interval_interval |
| Interval × Point | Reduced Allen subset: point-before-interval, point-during-interval, point-after-interval | temporal_operators.yaml §interval_point |
| Point × Point | Temporal order + Δt histogram only (before(Δt) or simultaneous(ε)) | temporal_operators.yaml §point_point |

Epsilon registry (config/temporal_operators.yaml):

epsilon_multiplier: 2.0

defaults:

interval_interval:

epsilon_seconds: 3600        # max(DCS_resolution, DCS_resolution) × 2

interval_point:

epsilon_seconds: 86400       # max(DCS_resolution, CMMS_resolution) × 2

point_point:

epsilon_seconds: 3600

per_source_pair_overrides:

# Override for coarse-grained maintenance pairs

- source_a: CMMS_WO

source_b: CMMS_WO

epsilon_seconds: 172800      # 2 days

Epsilon is not global. Per-source-pair overrides handle mismatched granularity between DCS (seconds) and maintenance records (days).

### 7.3 Temporal Score Composition

The temporal confidence dimension is a weighted composite of two components:

# Component 1: TSKR partial-order score

# Applies when TSKR pattern exists with n_cases >= config.min_tskr_cases (default 5)

tskr_score = max(0, 1 − |observed_lag − tskr_delta_t_mean| / (2 × tskr_delta_t_std))

# 1.0 when observed lag = mean; 0.0 when lag deviates by >2σ

# Component 2: FMEA latency window check

# Applies in all cases; sole component when TSKR history insufficient

σ_event  = timestamp_precision_hours(event.trigger_source)

# alarm=0.25h, operator_observation=1.0h, surveillance_test=0.5h,

# scheduled_inspection=4.0h, other=2.0h

σ_FM     = (FM.latency_max_days − FM.latency_min_days) × 24 / 4

σ_combined = sqrt(σ_event² + σ_FM²)

expected_midpoint = E_h.t_end + (latency_min + latency_max)/2 × 24   [hours]

expected_range    = (latency_max − latency_min) × 24                  [hours]

distance          = |E_now.t_start − expected_midpoint|

fmea_latency_score = max(0, 1 − distance / (expected_range/2 + σ_combined))

# Composite temporal score

if tskr_n_cases >= config.min_tskr_cases:

temporal_score = 0.70 × tskr_score + 0.30 × fmea_latency_score

else:

temporal_score = fmea_latency_score   # full fallback to FMEA check

# fallback_used = true logged in tskr_patterns.insufficient_history_pairs

Maintenance factor candidates use Point×Point operator family. FMEA latency does not apply. Their temporal score uses Δt histogram from TSKR point_point mining only:

temporal_score_maintenance = tskr_score (point_point)   if n_cases >= threshold

= 0.5 (neutral)               otherwise

### 7.4 Precursor Library (Proactive Mode)

The precursor library is a derived KG layer built from accumulated TSKR mining results. After each accepted RCA write-back, the TSKR chain leading to the confirmed failure mode is extracted and merged into the (:PrecursorPattern) node for that fm_id.

Each PrecursorPattern records:

- The ordered TSKR event-type chain (e.g., LUBE_DEGRADATION → BEARING_TEMP_ANOMALY → VIBRATION_HIGH → FAILURE)

- Mean and standard deviation of inter-event lags at each step

- Number of historical cases supporting each link

Proactive use: the monitoring loop queries the precursor library against the current live event stream. If an asset's recent validated anomalies match the early steps of a known precursor chain, the system surfaces a proactive alert — "this asset appears to be at step 2 of 4 in the FM-BEAR-WEAR-001 precursor pattern, estimated 3–7 days from failure" — without waiting for an explicit triggering event.

### 7.5 Common-Cause Failure Handling

TSKR simultaneous labels directly address CCF detection. If two or more events on different assets share the same fm_id and carry TSKR_SIMULTANEOUS edges to each other, the orchestrator flags both with ccf_flag: true in causality_candidates.json. Full CCF modeling remains a planned extension.

## 8. Algorithms & Pseudocode

### 8.1 Step 1 — Event Framing

function frame_event(event_record, config):

validate_required_fields(event_record)

Δt_table = {

"CRITICAL": {"pre": 96, "post": 48},

"HIGH":     {"pre": 72, "post": 24},

"MEDIUM":   {"pre": 48, "post": 24},

"LOW":      {"pre": 24, "post": 12}

}

Δt = Δt_table[event_record.severity]

event_record.analysis_window = {

"start": event_record.timestamp_start − Δt["pre"]  hours,

"end":   event_record.timestamp_end   + Δt["post"] hours

}

return event_record   # → event.json

### 8.2 Step 2 — Telemetry Snapshot (Matrix Profile + MMD)

function build_telemetry_snapshot(event, sensors, config):

window   = event.analysis_window

baseline = config.baseline_window

results  = []

for sensor in sensors:

ts = telemetry_api.get(sensor, window)

if ts is empty:

results.append({sensor_id: sensor, no_data: true})

continue

# --- Stage A: Matrix Profile anomaly detection ---

mp = compute_matrix_profile(

ts,

window_size = config.mp_window_size,   # configurable; default 288 (1-day @ 5-min)

seed        = config.random_seed

)

anomaly_intervals = extract_discords(mp, threshold=config.mp_discord_threshold)

motif_intervals   = extract_motifs(mp, threshold=config.mp_motif_threshold)

# --- Stage B: MMD validation of each matrix-profile anomaly ---

validated_anomalies = []

for anomaly in anomaly_intervals:

l_front = ts[window.start : anomaly.t_in]

l_dur   = ts[anomaly.t_in : anomaly.t_end]

l_rear  = ts[anomaly.t_end : window.end]

mmd_stat, p_value = compute_mmd(l_front, l_dur, kernel=config.mmd_kernel)

effect_size       = compute_effect_size(l_front, l_dur)

anomaly.mmd_validated = (p_value < config.mmd_alpha)

anomaly.mmd_statistic = mmd_stat

anomaly.p_value       = p_value

anomaly.effect_size   = effect_size

# Promotion gate: MP score + MMD validation required

anomaly.promoted_to_kg_event = (

anomaly.severity_score >= config.mp_promotion_threshold

AND anomaly.mmd_validated

)

if anomaly.promoted_to_kg_event:

anomaly.promotion_rationale = build_rationale(anomaly)

anomaly.kg_event_id = kg.create_abnormal_event_node(anomaly, event)

validated_anomalies.append(anomaly)

# --- Stage C: Baseline stats ---

ts_baseline = telemetry_api.get(sensor, baseline)

stats       = compute_descriptive_stats(ts)

baseline_comp = compare_to_baseline(ts, ts_baseline)

results.append({

sensor, stats, baseline_comp,

mp_results: {mp, anomaly_intervals, motif_intervals},

validated_anomalies,

data_quality: assess_data_quality(ts)

})

return build_telemetry_summary(event, results)   # → telemetry_summary.json

### 8.3 Step 3 — KG Narrowing

function kg_narrow(event, config):

components = kg.query("""

MATCH (a:Asset {asset_id: $asset_id})-[:REALIZES]->(c:Component)

RETURN c

""", asset_id=event.asset_id)

neighbors        = expand_neighborhood(components,

depth=config.hop_limit,

min_path_strength=config.min_edge_weight)

failure_modes    = fetch_failure_modes(neighbors)

past_events      = fetch_past_events(neighbors,

within_days=config.past_event_window)

documents        = fetch_documents_in_neighborhood(neighbors)

upstream_paths   = compute_paths(neighbors, direction="upstream")

downstream_paths = compute_paths(neighbors, direction="downstream")

return kg_context(

subgraph_id         = generate_subgraph_id(event.event_id),

kg_snapshot_version = kg.get_current_snapshot_version(),

components, neighbors, failure_modes, past_events,

documents, upstream_paths, downstream_paths

)   # → kg_context.json

### 8.4 Step 3A — TSKR Temporal Pattern Mining

function mine_tskr_patterns(kg_context, validated_event_stream, config):

"""

Operates on the historical corpus of validated, typed events within the

KG subgraph neighborhood. Produces temporal labels for all candidate pairs.

"""

subgraph_events = filter_events_to_subgraph(

validated_event_stream,

kg_context.components,

lookback_days=config.tskr_lookback_days

)

# Build event source registry lookup for type tags

source_registry = kg.query("""

MATCH (e:Event)-[:FROM_SOURCE]->(ds:DataSource)

WHERE e.event_id IN $event_ids

RETURN e.event_id, ds.native_type, ds.timestamp_resolution_seconds

""", event_ids=[e.event_id for e in subgraph_events])

candidate_pairs = generate_candidate_pairs(

kg_context.upstream_paths,

kg_context.downstream_paths,

kg_context.failure_modes

)

patterns = []

insufficient = []

for (type_a, type_b) in candidate_pairs:

# Resolve operator family from type tags

native_a = source_registry[type_a].native_type

native_b = source_registry[type_b].native_type

operator_family = select_operator_family(native_a, native_b)

epsilon = get_epsilon(operator_family, type_a, type_b, config.epsilon_registry)

# Collect historical instances of this pair

instances_a = [e for e in subgraph_events if e.event_type == type_a]

instances_b = [e for e in subgraph_events if e.event_type == type_b]

pairs = find_co_occurring_pairs(instances_a, instances_b,

epsilon=epsilon,

operator_family=operator_family)

if len(pairs) < config.min_tskr_cases:

insufficient.append({type_a, type_b, n_cases_available: len(pairs)})

continue

# Classify: simultaneous or partial order?

overlap_count = count_overlapping(pairs, epsilon)

order_count   = count_consistently_ordered(pairs)

if overlap_count / len(pairs) >= config.simultaneous_threshold:

label = "simultaneous"

delta_t_stats = None

elif order_count / len(pairs) >= config.partial_order_threshold:

label = "precedes"

delta_t_stats = compute_lag_statistics(pairs)

else:

label = "unordered"

delta_t_stats = None

pattern = build_tskr_pattern(type_a, type_b, label, delta_t_stats,

len(pairs), operator_family, epsilon)

patterns.append(pattern)

# Annotate KG edges with TSKR label

kg.merge_edge(

from_node = type_a, to_node = type_b,

edge_type = f"TSKR_{label.upper()}",

properties = delta_t_stats

)

# Update precursor library for confirmed failure-mode chains

if label == "precedes" and is_fm_precursor_chain(type_a, type_b, kg_context):

kg.merge_precursor_pattern(type_b, pattern)

return tskr_patterns(patterns, insufficient)   # → tskr_patterns.json

### 8.5 Step 4 — Causality Candidate Generation

function propose_causality(event_now, kg_context, tskr_patterns,

telemetry_summary, config):

candidates = []

# --- Coincidence pre-pass: collapse co-symptoms ---

simultaneous_pairs = [p for p in tskr_patterns.patterns

if p.tskr_label == "simultaneous"]

compound_symptoms = collapse_coincident_pairs(

simultaneous_pairs,

telemetry_summary.validated_anomalies

)

# Coincident event types are now grouped as a single compound symptom entry.

# They do not generate separate candidates.

# --- Hypothesis type 1: Failure Mode candidates ---

for fm in kg_context.failure_modes:

if not pattern_compatible(fm, telemetry_summary, compound_symptoms): continue

path    = best_path(fm, event_now, kg_context)

tskr_p  = get_tskr_pattern(fm, event_now, tskr_patterns)

temporal = compute_temporal_score(tskr_p, fm, event_now, telemetry_summary, config)

telemetry_s = compute_telemetry_score(fm, telemetry_summary)

scores = {

structural: compute_path_strength(path),

evidence:   evidence_score(fm, kg_context, evidence_bundle),

temporal:   temporal,

telemetry:  telemetry_s,

governance: governance_score(fm, pm_compliance)

}

candidates.append(build_candidate("failure_mode", fm, scores, path, tskr_p))

# --- Hypothesis type 2: Component fault candidates ---

for component in kg_context.components:

if not has_mmd_validated_anomaly(component, telemetry_summary): continue

path   = best_path(component, event_now, kg_context)

tskr_p = get_tskr_pattern(component, event_now, tskr_patterns)

scores = {

structural: compute_path_strength(path),

evidence:   evidence_score(component, kg_context, evidence_bundle),

temporal:   compute_temporal_score(tskr_p, None, event_now,

telemetry_summary, config),

telemetry:  component_telemetry_score(component, telemetry_summary),

governance: governance_score(component, pm_compliance)

}

candidates.append(build_candidate("component_fault", component, scores, path, tskr_p))

# --- Hypothesis type 3: Procedural deviation candidates ---

for doc in kg_context.documents_in_neighborhood:

if doc.doc_type not in ["SOP","CR"]: continue

deviations = extract_procedural_deviations(doc, evidence_bundle)

if len(deviations) < config.min_procedural_corroboration: continue

tskr_p = get_tskr_pattern_for_doc(doc, event_now, tskr_patterns)

scores = {

structural: sop_structural_score(doc, kg_context),

evidence:   len(deviations) / config.max_deviation_signals,

temporal:   compute_temporal_score(tskr_p, None, event_now,

telemetry_summary, config),

telemetry:  0.3,

governance: governance_score_procedural(doc, pm_compliance)

}

candidates.append(build_candidate("procedural_deviation", doc, scores, None, tskr_p))

# --- Hypothesis type 4: Maintenance factor candidates ---

# Uses Point×Point operator family; no FMEA latency fallback

for wo in kg_context.documents_in_neighborhood:

if wo.doc_type != "WO": continue

if not recent_maintenance_relevant(wo, event_now, config.maintenance_window_days): continue

tskr_p = get_tskr_pattern_point_point(wo, event_now, tskr_patterns)

temporal = tskr_p.tskr_score if tskr_p and tskr_p.n_cases >= config.min_tskr_cases \

else 0.5   # neutral fallback for maintenance events

scores = {

structural: maintenance_structural_score(wo, event_now, kg_context),

evidence:   evidence_score(wo, kg_context, evidence_bundle),

temporal:   temporal,

telemetry:  0.3,

governance: pm_compliance.summary.compliance_rate

}

candidates.append(build_candidate("maintenance_factor", wo, scores, None, tskr_p))

# --- Score, filter, rank ---

for c in candidates:

c.composite_score          = aggregate_score(c.scores, scoring_config)

c.meets_evidence_threshold = (

count_nonzero_dimensions(c.scores) >= scoring_config.min_dimensions_above_zero

AND c.composite_score >= scoring_config.min_composite_score

)

candidates = [c for c in candidates if c.meets_evidence_threshold]

candidates.sort(key=lambda c: c.composite_score, reverse=True)

flag_common_cause_candidates(candidates, config)

return causality_candidates(candidates, scoring_config)

# → causality_candidates.json  ⟹ HUMAN CHECKPOINT GATE 1

### 8.6 Step 5 — Evidence Retrieval

(Unchanged from v2. Scope strictly bounded to kg_context.documents_in_neighborhood.)

### 8.7 Step 6 — Ishikawa Evaluation

(Unchanged from v2.)

### 8.8 Step 7 — Effectiveness & Compliance

(Unchanged from v2.)

### 8.9 Step 8 — LLM Synthesis

LLM inputs now include TSKR pattern summaries (as a structured block: pattern_id, label, n_cases, Δt statistics, interpretation string) alongside the telemetry summary, KG paths, Ishikawa narrative, and evidence snippets. TSKR patterns are citable sources in rca_card.citations[] with source_type: "tskr_pattern".

Validation gate unchanged from v2: schema validation + causal sentence citation check.

### 8.10 Steps 9–10 — Analyst Review & Write-back

On ACCEPT, in addition to v2 write-back operations:

- TSKR_PRECEDES and TSKR_SIMULTANEOUS edges in the subgraph are committed to the permanent KG (they previously existed only in tskr_patterns.json).

- The confirmed precursor chain is merged into the :PrecursorPattern node for the confirmed fm_id, incrementing n_cases.

## 9. Implementation Notes

### 9.1 Module Structure

dackar/

├── config/

│   ├── kg_weights.yaml

│   ├── confidence_rules.yaml

│   ├── telemetry_windows.yaml

│   ├── temporal_operators.yaml     # Operator families + epsilon registry

│   ├── ishikawa_checks.yaml

│   └── ishikawa_interactions.yaml

├── spec/

│   ├── method_spec.md

│   ├── schemas/

│   │   ├── event.json

│   │   ├── telemetry_summary.json

│   │   ├── kg_context.json

│   │   ├── tskr_patterns.json      # New in v3

│   │   ├── causality_candidates.json

│   │   ├── evidence_bundle.json

│   │   ├── ishikawa_matrix.json

│   │   ├── pm_compliance.json

│   │   ├── operational_context.json

│   │   └── rca_card.json

│   └── examples/

├── src/

│   ├── orchestrator/

│   ├── ingest/

│   │   └── entity_resolution/      # Fuzzy matching to canonical asset_id

│   ├── kg/

│   ├── telemetry/

│   │   ├── matrix_profile/         # MP anomaly detection + motif extraction

│   │   └── mmd/                    # MMD validation (event × signal)

│   ├── temporal/

│   │   └── tskr/                   # TSKR mining, operator selection, epsilon registry

│   ├── causal/

│   ├── evidence/

│   ├── ishikawa/

│   ├── synthesis/

│   └── ui/

└── tests/

├── unit/

├── integration/

└── regression/

└── golden/

### 9.2 Human Checkpoint Gates

Two formal gates (unchanged from v2): PAUSED_GATE_1 after causality_candidates.json; PAUSED_GATE_2 after rca_card.json. All gate decisions are signed and written to the immutable audit log.

## 10. Validation, Tests & Acceptance Criteria

### 10.1 Unit Tests (additions for v3)

| Test | Description |
| --- | --- |
| test_mp_anomaly_detection | Synthetic signal with injected discord → assert correct t_in, t_end, severity_score |
| test_mp_motif_extraction | Synthetic signal with injected recurring pattern → assert motif detected |
| test_mmd_validation | Known distributional shift → assert validated=true, p < alpha; no shift → validated=false |
| test_mmd_no_shift | Null case — signal unchanged around event → assert anomaly not promoted |
| test_tskr_operator_selection | Each pair type (II, IP, PP) → assert correct operator family selected |
| test_tskr_simultaneous | Synthetic co-occurring event pairs → assert simultaneous label and coincidence collapse |
| test_tskr_precedes | Synthetic ordered event pairs → assert precedes label, correct Δt statistics |
| test_tskr_insufficient_history | n_cases < threshold → assert fallback to FMEA latency, fallback_used=true |
| test_epsilon_registry | Source pair with day-resolution CMMS events → assert per-pair epsilon applied |
| test_temporal_score_composition | TSKR + FMEA inputs → assert correct weighted composite |
| test_maintenance_temporal_score | Point×Point maintenance pair → assert FMEA latency not used |
| test_coincidence_collapse | Two simultaneous anomalies → assert single compound symptom, no duplicate candidates |
| test_precursor_library_update | Accepted RCA write-back → assert PrecursorPattern n_cases incremented |
| test_tskr_citation_in_rca_card | TSKR pattern cited in narrative → assert source_type=tskr_pattern in citations |

### 10.2 Integration Tests

- Full pipeline with TSKR: Synthetic event corpus with injected PRECEDES pattern → full pipeline → assert causality_candidates temporal score reflects TSKR evidence, not FMEA fallback.

- Coincidence suppression: Two simultaneous anomalies on same asset → assert one compound candidate, not two separate ones.

- Proactive alert: Live event matches steps 1–2 of a precursor pattern → assert proactive alert generated before failure event.

- Fallback path: Asset with sparse history (n_cases < threshold) → assert fallback_used=true and FMEA latency score applied.

### 10.3 Regression & Acceptance Criteria

(Unchanged from v2.) Golden baseline in tests/regression/golden/ includes tskr_patterns.json as a required regression artifact in v3.

## 11. Security, Privacy & Regulatory Considerations

(Unchanged from v2 — 10 CFR 50 Appendix B, 10 CFR 50.59, EPRI Cause Analysis Manual alignment, LLM containment, audit log immutability.)

## 12. Example Walkthrough — Pump P-101A Bearing Failure

- Event framing: E2026-01-01-001 on P-101A, HIGH severity. Window: 72h pre / 24h post.

- Matrix profile: Gradual drift discord on S-T-101 at index 4312, severity 0.83. Anomaly interval ANO-MP-001: 2025-12-27 to 2026-01-01. Recurring 48h motif from November identified (potential precursor motif).

- MMD validation: l_front vs. l_dur MMD test: p=0.003 (< α=0.05), large effect size. ANO-MP-001 validated. Promoted to KG event KGE-2026-0001.

- KG narrowing: Subgraph SG-E2026-01-01-001 (3 hops). Components: C-BEARING-101A, C-LUBE-SYS, C-SHAFT-101A. FM: FM-BEAR-WEAR-001. Past event E-2025-09-17-003 (106 days prior).

- TSKR mining: VIBRATION_HIGH ↔ TEMP_ELEVATED → simultaneous (n=9) — collapsed to compound symptom. LUBE_DEGRADATION → BEARING_TEMP_ANOMALY → precedes (n=7, Δt=108±36h). KG edges annotated.

- Causality candidates: FM-BEAR-WEAR-001 composite 0.81. Temporal score: 0.87 (TSKR 0.91 × 0.70 + FMEA 0.85 × 0.30). Vibration and temperature not separate candidates — collapsed. Gate 1: analyst approves.

- Evidence retrieval: WO-2025-4821 (inner race scoring), CR-2026-0042 (lubricant discoloration), SOP-LUBE-012 (viscosity spec). All within subgraph scope.

- Ishikawa: Methods (0.80), Materials (0.75), Management (0.70). Methods×Materials amplification 1.40.

- LLM synthesis: Narrative cites TSKR-002 directly: "lubrication degradation events have preceded bearing temperature anomalies by ~4.5 days across 7 historical cases on similar equipment." validation_status.schema_valid=true, all_claims_cited=true.

- Gate 2 + write-back: Analyst accepts. CONFIRMED_CAUSE edge written. TSKR_PRECEDES edge committed to permanent KG. PrecursorPattern for FM-BEAR-WEAR-001 updated: n_cases incremented to 8.

## 13. Deliverables

| Item | Path |
| --- | --- |
| Method specification | spec/method_spec.md |
| JSON Schemas (10 files, incl. tskr_patterns.json) | spec/schemas/*.json |
| Config files (incl. temporal_operators.yaml) | config/*.yaml |
| Synthetic example | spec/examples/synthetic_bearing_wear/ |
| Module scaffolding (incl. telemetry/matrix_profile/, telemetry/mmd/, temporal/tskr/) | src/*/ |
| Unit + integration + regression test stubs | tests/ |
| README | README.md |