## Slide: Problem Statement

Root cause analysis in nuclear systems is not simply a data analysis problem; it is fundamentally a problem of causal ambiguity under imperfect and heterogeneous information. Engineers are required to interpret telemetry trends, maintenance records, and system relationships simultaneously, often under time pressure. The difficulty is not just in accessing data, but in determining how different observations relate causally.

In practice, systems are tightly coupled. A disturbance in one subsystem can propagate across multiple others, producing symptoms that appear far from the initiating cause. This leads to a common challenge: the most visible signal is often not the root cause. Engineers must mentally reconstruct system behavior across multiple domains to determine what actually initiated the event.

Another major challenge is temporal ambiguity. Signals may occur close together or overlap, making it difficult to determine which one is causal. Without a structured approach, it is easy to confuse cause and effect, especially in systems with feedback loops or delayed responses.

Additionally, information is scattered across multiple sources—CRs, WOs, telemetry, and diagrams—each with its own format and context. Engineers spend a disproportionate amount of time assembling this information before actual reasoning can begin. This contributes to variability in RCA outcomes and reliance on individual experience rather than consistent methodology.

## Slide: Project Goal

The objective is to provide a structured and explainable RCA methodology that supports engineers without replacing their judgment. The system is explicitly designed as a co-pilot, reinforcing disciplined reasoning rather than automating conclusions. This is critical in a nuclear context, where decisions must remain under human control.

The methodology enforces a fundamental RCA principle: generate hypotheses first, then test them against evidence. This prevents confirmation bias, where conclusions are formed prematurely and evidence is interpreted selectively to support them.

A key aspect of the approach is the separation of reasoning types:

- structural (what is physically possible),

- temporal (what happens first),

- evidence-based (what has been observed historically).

By separating these, engineers can independently validate each dimension, ensuring that conclusions are not driven by a single misleading signal or dataset.

The system also explicitly represents:

- contradictions,

- weak or incomplete evidence,

- alternative hypotheses,

- and “review_required” decisions.

This ensures that uncertainty is visible and managed, aligning with conservative engineering practice.

## Slide: Data Sources and Data Representation

The methodology integrates multiple data sources, each contributing a distinct perspective. Telemetry data captures real-time system behavior, including anomalies and trends. Plant documents provide historical insights into similar events and corrective actions. The knowledge graph represents system structure and functional relationships.

Operational context and maintenance data provide additional information about system conditions and recent interventions. These inputs are critical for interpreting whether observed behavior is expected, anomalous, or influenced by external factors.

A central principle is that no single data source is trusted alone. Telemetry may be noisy or affected by sensor issues. Documents may reflect historical bias or incomplete understanding. Structural models may not capture transient behavior. Causality is inferred only when multiple sources converge.

Data reliability is explicitly acknowledged. Issues such as timestamp misalignment, sensor drift, and missing data are inherent to plant operations. The methodology ensures that reasoning remains robust despite these imperfections by relying on cross-validation across sources.

## Slide: Data entities: Knowledge Graph and Chroma

The knowledge graph serves as a functional representation of the plant, not just a data structure. It encodes components, relationships, and failure modes, allowing the analysis to be constrained to physically meaningful possibilities.

This structural context is essential because it prevents consideration of impossible or irrelevant causes. For example, a failure mode can only be considered if it is associated with a component that is connected to the affected system.

The Chroma-based retrieval system enables access to relevant historical information from plant documents. Queries are generated based on candidate hypotheses and system context, ensuring that retrieved information is directly relevant to the analysis.

However, retrieved evidence is not automatically accepted. It must align with both system behavior and timing. This ensures that conclusions are grounded in real plant history while maintaining critical evaluation.

## Slide: Solution: Multi-stage RCA reasoning pipeline

The RCA process is decomposed into a sequence of stages, each reducing uncertainty and refining understanding. This staged approach mirrors how experienced engineers think but enforces consistency and traceability.

The pipeline integrates three complementary reasoning dimensions:

- structure (how systems are connected),

- time (how signals evolve),

- evidence (what has been observed historically).

Each stage transforms raw data into higher-level understanding. Rather than jumping directly to conclusions, the process ensures that each reasoning step is explicit and can be independently validated.

Importantly, the system does not produce a single answer immediately. Instead, it generates ranked hypotheses with supporting evidence and reasoning traces, allowing engineers to evaluate and compare alternatives.

## Slide: RCA Workflow (overview)

The workflow transforms:
Event + telemetry + context → causal hypotheses → evidence-grounded explanation → RCA output

This transformation is achieved through a sequence of reasoning steps, each building on the previous one. The process is not a single model but a composition of reasoning layers.

The process begins with defining the event and extracting relevant telemetry. Structural context is then used to define what is physically possible. Temporal reasoning determines how signals relate in time, and candidate causes are generated and evaluated.

Evidence is then retrieved and linked to candidates, and the reasoning is structured into a final RCA output. Each step contributes to refining the understanding of the event.

This structured approach ensures that reasoning is transparent and reproducible, which is essential for engineering validation and regulatory defensibility.

## Slide: RCA Workflow (Stages 1–2)

Stage 1 focuses on what is possible. Using the knowledge graph, the system identifies relevant components, relationships, and failure modes. This defines the scope of analysis and ensures that only physically plausible hypotheses are considered.

Stage 2 focuses on temporal consistency. Telemetry data is analyzed to determine the sequence of events. Signals are classified based on whether they occur before, during, or after the main event.

Temporal reasoning is critical for distinguishing cause from effect. Signals that precede the event are potential causes, while those that follow are likely consequences. Simultaneous signals are treated as ambiguous.

Additional metrics, such as time lag and consistency across signals, are used to strengthen temporal conclusions. This ensures that timing-based reasoning is robust and not based on isolated observations.

## Stage 1 — KG Context Construction: What is physically possible?

At this stage, the problem is constrained to a physically meaningful subset of the plant. The system performs a graph traversal over the knowledge graph, starting from the asset and seed components associated with the event. This traversal identifies all components within a defined connectivity radius (e.g., upstream/downstream paths, support systems).

The underlying algorithm is a bounded graph expansion:

- nodes = components, systems, failure modes

- edges = process flow, support relationships, dependencies

- traversal = breadth-first or constrained multi-hop search

This produces a subgraph representing the relevant causal domain. Importantly, this step removes irrelevant parts of the plant, preventing spurious correlations from being considered later.

Failure modes are then attached to each component using a component-to-failure-mode mapping. This defines the initial hypothesis space. Past events and documents are also associated with this subgraph, providing historical context.

The key principle here is: Causality must be structurally plausible before it is considered.

Stage 2 — Temporal Reasoning (TSKR): Does timing support causality?

Temporal reasoning evaluates the ordering and consistency of observed anomalies.

The system extracts anomaly timestamps from telemetry and applies a temporal relation classification:

- precedes → candidate cause

- simultaneous → ambiguous

- follows → likely effect

This classification is derived using interval/point temporal logic, comparing anomaly windows and event timing. Additional metrics are computed:

- mean lag between signals

- standard deviation of lag (consistency)

- anomaly count (strength of evidence)

These are combined into a temporal support score using a weighted function.

Conceptually, this is a form of constraint-based temporal inference:

- causality requires temporal precedence

- consistency across signals strengthens confidence

The key principle is: A cause must occur before its effect, and consistently so.

## Slide: RCA Workflow (Stages 3–4)

Stage 3 generates candidate causes from physically plausible failure modes and past events. Each candidate is evaluated across multiple dimensions, including structural, temporal, telemetry, evidence, and governance factors.

This multi-dimensional evaluation ensures that candidates are not ranked based on a single metric. Instead, each hypothesis is assessed holistically, reducing the risk of misleading correlations.

Stage 4 retrieves relevant evidence from plant documents. Snippets are selected based on their relevance to candidate hypotheses and system context.

Each snippet is classified as supporting, contradicting, or contextual. This classification is essential for understanding how evidence contributes to or challenges each hypothesis.

Stage 3 — Candidate Generation (Causality Engine): What are plausible causes?

Candidates are generated from:

- failure modes in the KG

- past events with similarity to current conditions

Each candidate is evaluated using a multi-criteria scoring model.

The scoring dimensions are:

- Structural score

  - Based on graph connectivity and path relevance

  - Higher if the failure mode is directly connected to affected components

- Temporal score

  - Derived from TSKR outputs

  - Reflects timing consistency

- Telemetry score

  - Measures alignment between observed anomalies and expected behavior of the failure mode

  - Essentially a pattern-matching function

- Evidence prior

  - Initial assumption of plausibility based on document availability

- Governance score

  - Reflects maintenance context and known issues

These scores are combined into a composite score using a weighted linear model.

This stage is essentially a hypothesis ranking problem under multiple constraints.

Stage 4 — Evidence Retrieval: What evidence exists?

Evidence retrieval is performed using vector similarity search over processed document embeddings.

Queries are constructed from:

- candidate failure modes

- component context

- system relationships

The retrieval algorithm uses:

- semantic similarity (embedding distance)

- metadata filters (component, asset, document type)

- query weighting (candidate vs context queries)

Returned snippets are then normalized into structured evidence entries. This is not just search—it is targeted evidence gathering, where each query is tied to a specific hypothesis.

The key principle is: Evidence must be relevant to the hypothesis being evaluated.

## More on stage 3 and 4

Think of it like a medical diagnosis. A doctor first generates a list of possible diagnoses based on the patient's symptoms and known disease mechanisms — without yet ordering any tests. That's Stage 3. Then the doctor orders specific lab tests to confirm or rule out each possibility. That's Stage 4. The two steps are deliberately separate because mixing them produces worse reasoning than doing them sequentially.

## Stage 3 — Candidate Generation: What could have caused this?

This stage asks: given what we know about the plant's physical structure, the telemetry anomalies, and the timing of signals, what are the plausible causal hypotheses?

What it reads as input:

- The KG context — specifically the list of failure modes and past events that were pulled for this asset

- The telemetry summary — anomaly counts, severity scores, signal timing

- The TSKR patterns — temporal relations (precedes/simultaneous/follows) and latency alignment scores

What it does NOT read: any plant documents, CRs, WOs, or SOPs. Those come later.

What it produces for each candidate:

For every failure mode in the KG neighborhood, it computes five independent scores:

Structural score — is this failure mode physically connected to the affected component? A failure mode that applies directly to the seed component scores high. One that requires traversing several hops in the KG scores lower.

Temporal score — does the timing support this mechanism? This combines the TSKR relation type (precedes scores highest), the latency alignment (did the anomaly appear within the expected time window for this failure mode?), and the anomaly burden. If the expected latency for a failure mode is 48–336 hours and the observed drift developed over 336 hours, the latency alignment score is high. If the expected latency is 2–48 hours but the drift took 336 hours, the latency consistency score drops and a temporal contradiction flag is set.

Telemetry score — are there enough anomalies, and are they severe enough, to be consistent with this failure mode? This counts anomaly instances and weights them by severity. It also checks whether the anomalous sensor maps to a component linked to this failure mode.

Evidence prior score — before we retrieve any documents, does the document neighborhood even contain relevant document types? If the KG context includes CRs, WOs, and FMEAs for this component, the prior score is higher than if only SOPs are present. This is not evidence retrieval — it's just asking whether the right types of documents exist in scope.

Governance score — does the PM compliance picture suggest a maintenance-related contributor? Currently this is an asset-level signal (failed PM checks on the asset boost all candidates uniformly), which is a known limitation.

These five scores are combined using fixed weights into a composite score. Candidates are then sorted by composite score, threshold-filtered (candidates below minimum composite or minimum evidence thresholds are removed), and truncated to top-k.

What comes out: a ranked list of candidate hypotheses, each carrying its composite score, its five component scores, rationale text for each score dimension, temporal evidence details, and in v32, recurrence and common-cause enrichment. No document snippets. No retrieved text. Just scored hypotheses based on structure, timing, and telemetry.

## Stage 4 — Evidence Retrieval: What does the historical record say about each hypothesis?

This stage asks: given the candidates we already ranked, what does the actual document corpus say that supports or contradicts each one?

What it reads as input:

- The top candidates from Stage 3 (specifically their cause_label, candidate_id, and hypothesis_type)

- The KG document list (which documents are in scope for this asset)

- The event context (asset_id, component_ids)

What it does NOT do: score candidates. It has no access to composite scores and performs no ranking of hypotheses. It only retrieves and classifies text.

What it produces:

For each top candidate, the retriever builds multiple targeted queries — not one generic search. For a candidate like "air in-leakage through expansion joint" it builds:

- A support query: "U2-CONDENSER-MAIN air in-leakage through boundary failure_mode" — looking for documents that describe this mechanism

- A contradiction query: "U2-CONDENSER-MAIN air in-leakage inspection not no normal failed to reproduce" — deliberately looking for documents that say the opposite

It also builds context queries for the failure mode class, the specific component, and the operational context.

Each retrieved snippet then goes through a role classifier that reads the snippet text and decides whether it is supporting, contradicting, or contextual for the candidate it was retrieved against. This classification uses keyword cues — phrases like "caused by," "resulted in," "degraded" push toward supporting; phrases like "within normal limits," "no evidence of," "failed to reproduce" push toward contradicting.

The output is an evidence bundle that contains two things: the raw ranked snippets, and a per-candidate summary that tells Stage 6 how many supporting, contradicting, and contextual snippets exist for each candidate, and what the best support and contradiction scores were.

## The critical difference between the two stages

|  | Stage 3 | Stage 4 |
| --- | --- | --- |
| Question | What could have caused this? | What does the record say about each candidate? |
| Input | KG structure, telemetry, TSKR timing | Candidate labels, document corpus |
| Produces | Scored hypothesis ranking | Classified evidence snippets per candidate |
| Uses documents? | No — only document type presence as a prior | Yes — full text retrieval and classification |
| Changes candidate scores? | Yes — this is where scores are first computed | No — classification only, no rescoring |
| Drives by | Physics, timing, anomaly burden | Historical records, procedure text, prior CRs |
| Output artifact | causality_candidates | evidence_bundle |

## Why they must be separate

If you merged them — retrieving evidence and scoring candidates simultaneously — you would lose the ability to detect when the evidence contradicts the structurally plausible hypothesis. The whole value of Stage 6 (refinement) comes from the fact that Stage 3 produced a ranking based purely on physics and timing, and Stage 4 can now move that ranking up or down based on what the documents actually say.

In the condenser test case, Stage 3 might rank air in-leakage and tube fouling very close together because both produce gradual drift in backpressure and both have plausible structural paths. Stage 4 then retrieves WO-2024-11847, which says "within normal limits, zero tubes plugged, no evidence of fouling" — and classifies that as contradicting evidence for the fouling candidate. Stage 6 takes that contradiction and drops the fouling candidate's evidence score, widening the gap between the two hypotheses. That sequence only works because the pre-evidence ranking and the evidence retrieval were kept separate.

## Slide: RCA Workflow (Stages 5–6)

Stage 5 evaluates the relationship between evidence and each candidate. Support, contradiction, and context scores are computed, and an overall evidence posture is derived.

This step is critical because it determines whether a hypothesis is truly supported by available information. Evidence must be consistent with both system behavior and timing to be considered valid support.

Stage 6 updates candidate scores based on evidence. This reflects the iterative nature of RCA, where hypotheses are refined as new information becomes available.

Candidates are filtered based on thresholds, but the methodology ensures that at least one meaningful alternative is retained when appropriate. This supports validation and prevents premature convergence.

Stage 5 — Evidence Adjudication: Does evidence support the hypothesis?

Each retrieved snippet is evaluated against a candidate using a heuristic scoring function.

Three scores are computed:

- support score

- contradiction score

- context score

These are based on:

- keyword/semantic alignment with the hypothesis

- presence of causal language

- consistency with system context

A classification rule assigns:

- supporting

- contradicting

- contextual

The scores are then aggregated into an evidence posture:

- supported

- mixed

- contextual_only

- weak

- contradicted

This is essentially a lightweight evidence inference model, approximating how engineers interpret documentation.

Stage 6 — Candidate Refinement: Update hypotheses using evidence

Candidates are rescored using an evidence-weighted update function:

refined_evidence = 
    w_prior * prior 
  + w_support * support 
  + w_context * context 
  - w_contradiction * contradiction

The composite score is then recomputed.

Candidates are filtered using:

- minimum evidence threshold

- minimum composite threshold

A key addition is the review alternative retention rule:

- if only one candidate passes thresholds

- retain the next-best candidate if it is close in score

This prevents premature convergence.

This stage implements: Bayesian-like update without full probabilistic modeling

## Slide: RCA Workflow (Stages 7–9)

Stage 7 evaluates recurrence and common-cause mechanisms. This provides context for understanding whether the event is part of a broader pattern or a systemic issue.

Stage 8 organizes reasoning into an Ishikawa structure, mapping causes into categories such as equipment, process, and measurement. This aligns with established RCA methodologies and supports engineer workflows.

Stage 9 synthesizes the results into a structured RCA output. This includes hypotheses, evidence, uncertainties, and recommended actions.

The output is explicitly marked as requiring review, ensuring that engineers remain responsible for final decisions.

Stage 7 — Recurrence and Common Cause Reasoning: Is this part of a pattern?

Recurrence is computed using a similarity-based matching algorithm:

- match current candidate to past events

- compute similarity based on:

  - component overlap

  - failure mode

  - asset

Scores are aggregated into a recurrence score.

Common cause reasoning evaluates shared dependencies across candidates:

- shared upstream components

- shared support systems

- symptom convergence

A composite common-cause score is derived.

This stage approximates: pattern recognition and clustering over causal hypotheses

Stage 8 — Ishikawa Structuring Organize reasoning for engineers

Candidates and evidence are mapped into categories:

- equipment

- process

- measurement

- environment

This is a classification and grouping operation, not scoring.

The purpose is not computational optimization, but: alignment with human reasoning frameworks

This step translates machine reasoning into engineering mental models, making results easier to interpret and validate.

Stage 9 — RCA Synthesis: What should the engineer conclude?

The final stage assembles:

- primary hypothesis (highest-ranked)

- alternatives

- evidence

- uncertainties

- recommended actions

Confidence is calibrated using:

- evidence posture

- temporal posture

- score separation

- recurrence/common-cause signals

Decision status is derived using rule-based logic:

- insufficient evidence → hold

- contradictions → review

- otherwise → review_required

This stage is deterministic synthesis, not inference: it consolidates reasoning, it does not create new reasoning

## Slide: Initial test case: Setup

The test case involves loss of condenser vacuum and rising turbine backpressure. Telemetry shows a clear sequence: air ejector DP increases first, followed by vacuum degradation and then backpressure increase.

This sequence provides a strong basis for temporal reasoning. The early change in air removal system behavior suggests it may be involved in initiating the event.

The knowledge graph identifies relevant components and failure modes, including air in-leakage and condenser fouling. Documents provide additional context from past events and maintenance actions.

The goal of the test case is to demonstrate that the methodology does not rely on correlation alone but integrates timing, structure, and evidence.

## Slide: Initial test case: Temporal Reasoning and Causal Evaluation

Temporal analysis shows that the air ejector DP anomaly precedes vacuum degradation. This strongly supports the air removal system as a potential initiating mechanism.

Condenser fouling appears simultaneously with vacuum loss, making it a plausible but less definitive cause. Feedwater instability occurs later, indicating it is likely a consequence rather than a cause.

The methodology evaluates each hypothesis based on timing, physical plausibility, and evidence. Air in-leakage satisfies all three criteria, making it the leading hypothesis.

Other hypotheses are weaker because they are either temporally inconsistent, indirect, or lack supporting evidence.

## Slide: Evidence-Based Reasoning

Evidence confirms the temporal and physical analysis. Condition reports indicate that air removal DP increased before vacuum loss, and work orders identify degraded air ejector seals.

Contextual evidence, such as SOPs, provides background information but does not independently determine the outcome. The distinction between supporting and contextual evidence is critical.

The key principle is that evidence must explain both the physics and the timing of the event. Air in-leakage satisfies this requirement, while condenser fouling remains a plausible alternative.

The final result preserves both the primary hypothesis and the alternative, providing a clear path for validation through inspection and testing.

Roles of LLMs in RCA workflow

LLMs are NOT doing causal reasoning in the RCA workflow. They are used for translation, synthesis, and explanation—not inference.

That distinction is critical for engineer trust.

Where LLMs sit in your pipeline

Your workflow is fundamentally:

deterministic + structured reasoning → LLM-assisted explanation layer

1. LLM is NOT used for hypothesis generation

Your hypotheses come from:

- KG failure modes

- past events

- structural constraints

This is rule-based + graph-based generation, not LLM-driven.

- ensures physical plausibility

- avoids hallucinated causes

2. LLM is NOT used for causal scoring

All scoring is computed from:

- temporal alignment (TSKR)

- structural connectivity (KG)

- telemetry matching

- evidence scoring

This is algorithmic and traceable.

The LLM does NOT decide:

- what is the primary cause

- what is causal vs effect

- what is plausible

3. LLM IS used for synthesis

This is where it plays a role in Stage 9 (RCA synthesis):

- convert structured outputs into narrative

- generate explanations engineers can read quickly

- phrase:

  - why_primary

  - uncertainties

  - analyst questions

  - summaries

LLM translates “structured reasoning → human-readable explanation

4. LLM is also used for language-level tasks

Potential roles (some already in your design, some future):

a. Evidence interpretation (light)

- summarizing snippets

- extracting key statements

b. Knowledge retrieval (semantic search)

- improving query generation

- mapping symptoms → relevant documents

c. Analyst interaction layer (future)

- answering “why” questions

- exploring alternatives interactively

5. Why this separation is important

From an engineering standpoint, this is the key design decision:

Your system:

- uses deterministic reasoning for causality

- uses LLM for communication

NOT:

- using LLM to infer causality directly

This aligns perfectly with:

- interpretability requirements

- regulatory defensibility

- human primacy principles

⚠️ What would be risky (and you avoided it)

You did NOT do:

- “LLM reads telemetry → outputs root cause”

- “LLM ranks hypotheses”

- “LLM infers temporal causality”

Those would:

- be non-traceable

- be hard to defend

- violate engineering expectations

Best way to explain this to engineers

If you need a one-liner for your presentation:

“The system does the reasoning deterministically and uses the LLM only to explain the reasoning in engineering language.”

Or slightly more detailed:

“All causal logic—timing, structure, evidence—is computed explicitly. The LLM is only used to translate that into readable RCA narratives.”

Future role of LLMs (if you expand)

Where LLMs could add value safely:

- assisting hypothesis brainstorming (non-binding)

- summarizing large CAP datasets

- generating comparison explanations between candidates

- helping with documentation drafting

But always: LLM suggestions should be traceable back to structured evidence

Final takeaway

Your current architecture gets this exactly right:

- LLM = explanation layer

- Causality = deterministic pipeline