# 🎯 Demo Narrative — PWR Secondary Side RCA

## 1. Context (Set the scene)

“We’re looking at a pressurized water reactor operating at high load, around 96% power. The event occurs on the secondary side, specifically involving the condenser, feedwater system, and some supporting systems like HVAC.”

## 2. Timeline of the Event (Make it concrete)

Use this as your backbone — this is what makes the demo intuitive.

### 🕒 Event Timeline

13:10 — Early precursor

- Cabinet temperature in instrumentation cabinet (AC12) begins to rise

- No immediate impact on plant performance

13:20 — Event starts

- Unit is stable at high load

- No alarms yet

13:22 — First anomaly

- Air removal train differential pressure shows a step change
👉 Potential degradation in vacuum support system

13:24 – 13:26 — Core symptom develops

- Condenser vacuum begins to degrade

- Turbine backpressure starts rising

👉 This is the primary plant-level symptom

13:31 – 13:34 — Secondary effects appear

- Hotwell level begins oscillating

- Feedwater flow becomes unstable

👉 These are likely effects, not causes

~14:00 — Operators respond

- Performance degradation is clearly visible

- Investigation begins

## 3. What the System Sees (Translate to pipeline stages)

“This is what our RCA pipeline is doing behind the scenes.”

### Step A — Structure (KG Context)

- Identify relevant components:

  - condenser

  - air removal system

  - hotwell

  - feedwater control

  - HVAC support system

👉 This defines where to look

### Step B — Temporal Reasoning (TSKR)

The system detects patterns like:

- Air removal anomaly precedes vacuum degradation

- Vacuum degradation and backpressure rise track together

- Feedwater oscillations follow the main event

👉 This is critical: it distinguishes cause vs effect

### Step C — Candidate Generation

The system evaluates multiple hypotheses:

- Air in-leakage / air removal degradation

- Condenser fouling

- Feedwater control instability

- HVAC/support degradation

## 4. Candidate Ranking (Explain the result)

Present it simply:

### 🥇 1. Air In-Leakage / Air Removal Degradation (Score: 0.83)

- Occurs first in time

- Directly affects condenser vacuum

- Supported by CR evidence

- Strong temporal + structural alignment

👉 Most likely root cause

### 🥈 2. Condenser Fouling (Score: 0.76)

- Physically plausible

- Supported by maintenance history

- But timing is less directional

👉 Strong alternative

### 🥉 3. Feedwater Control Instability (Score: 0.59)

- Explains oscillations

- Happens after vacuum degradation

👉 Effect, not root cause

### 4. HVAC Support Degradation (Score: 0.60)

- Precursor exists (cabinet temperature)

- Indirect influence

👉 Contributing factor, not primary cause

## 5. Ishikawa View (Explain visually)

“This is how the system organizes causes like a fishbone diagram.”

### Categories:

Equipment / Hardware

- Air removal degradation

- Condenser fouling

- Feedwater control valve instability

Measurement / Instrumentation

- Vacuum signal anomaly

- Backpressure signal anomaly

- Flow oscillations

Process / Procedure

- CRs and WOs describing degradation trends

Maintenance / Human Factors

- Overdue PM tasks (air removal, HVAC)

Environment / Context

- High load operation

- Elevated cabinet temperature

👉 This shows multiple contributing factors, not just one cause.

## 6. Final RCA Card (What decision-makers see)

“The system synthesizes everything into a concise RCA output.”

Primary hypothesis:

Air in-leakage or degraded air removal performance caused condenser vacuum degradation.

Why:

- Correct temporal ordering (precedence)

- Strong structural connection

- Supported by evidence (CR)

- Consistent telemetry across multiple signals

Alternatives:

- Condenser fouling (credible but less temporally supported)

Actions:

- Inspect air removal train A

- Verify vacuum integrity and leak paths

- Check condenser cleanliness as secondary check

## 7. Key Takeaways (Close the demo)

End with this — it’s what people remember:

- The system does not just detect anomalies

- It reconstructs causality using structure + time + evidence

- It distinguishes:

  - cause vs effect

  - primary vs contributing factors

- It produces:

  - ranked hypotheses

  - structured reasoning (Ishikawa)

  - actionable recommendations