# Formal Method Specification — Root Cause Analysis (RCA) using Knowledge Graphs

## 1. Purpose & Scope

Purpose: define a formal, machine-readable, auditable method to perform RCA for nuclear power plant equipment using a knowledge graph (KG) built from MBSE and engineering repositories, telemetry, and maintenance/governance systems.

Scope: single-event RCA augmented by historical events and corpora. The method covers Steps 1–10 from your draft: event framing → telemetry snapshot → KG narrowing → KG-based causality → evidence retrieval → Ishikawa assessment → effectiveness review → LLM synthesis → analyst review → write-back. (See original workflow for high-level context.)

project_notes

Assumptions:

- KG (Neo4j) and vector store (Chroma or equivalent) are available.

- Telemetry store (historian/Spark) and CMMS/PLM/EAM are accessible via APIs.

- Users are system engineers with authority to make final decisions; LLM outputs are advisory.

## 2. Key Definitions (Formal)

- Event (E): E = {id, timestamp_start, timestamp_end, asset_id, severity, event_type, metadata}
event_type ∈ {SYMPTOM, DEGRADATION, INTERVENTION, FAILURE}

- Failure Mode (FM): structured entity describing how a component can fail. FM = {fm_id, name, superclasses[], mechanism[], detection_methods[], expected_latency}

- Component (C): physical or logical system element. C = {component_id, model_id, location_id, interfaces[], sensors[]}

- Telemetry Signal (T): time-series measurement. T = {asset_id, sensor_id, timestamp, value, unit, quality_flags}

- KG Path Strength (path_strength): numerical score describing how strongly two nodes are connected given semantics, edge weights, and path length.

- Confidence Object: multidimensional confidence for causal claim. See section 6.

- RCA Card: structured result containing hypotheses, evidence, actions, and provenance.

## 3. Overall Architecture (Components & Interfaces)

- Ingestion & Snapshot Module

  - Reads: Event stream, telemetry store, ops logs, CMMS/EAM, PLM.

  - Produces: telemetry_summary.json, ops_context.json.

- KG Query & Narrowing Module

  - Reads: Neo4j KG

  - Produces: kg_context.json (components, upstream/downstream, FMs, SOPs, FMEA entries, ECRs, past events)

- Causality Engine

  - Reads: telemetry_summary.json, kg_context.json

  - Produces: causality_candidates.json

- Evidence Retriever

  - Reads: kg_context.json → queries Chroma → produces evidence_bundle.json

- Ishikawa Evaluator

  - Inputs: telemetry, ops_context, evidence_bundle

  - Produces: ishikawa_matrix.json

- Effectiveness & Compliance Evaluator

  - Inputs: CMMS/EAM, LCM, PM data

  - Produces: effectiveness_compliance.json

- LLM Synthesis

  - Inputs: all above JSONs + snippets → produces rca_card.json + rca_card.md

- Analyst Review UI

  - Presents rca_card and allows accept/modify/reject. On accept, Write-Back Module commits nodes/edges to KG and stores RCA in Chroma.

External Interfaces (API endpoints):

- GET /telemetry?asset_id=&from=&to=

- GET /ops/logs?asset_id=&from=&to=

- POST /kg/query (Neo4j cypher wrapper)

- POST /chroma/query (vector search)

- POST /kg/commit (writes new edges/nodes)

- POST /rca_card (store final card)

## 4. Data Models & JSON Schemas (examples)

Below are compact schemas. In the repo, put them as JSON Schema files.

### 4.1 Event (E) — event.json

{

"type": "object",

"required": ["id","timestamp_start","timestamp_end","asset_id","severity"],

"properties": {

"id": {"type":"string"},

"timestamp_start": {"type":"string","format":"date-time"},

"timestamp_end": {"type":"string","format":"date-time"},

"asset_id": {"type":"string"},

"severity": {"type":"string","enum":["LOW","MEDIUM","HIGH","CRITICAL"]},

"event_type": {"type":"string","enum":["SYMPTOM","DEGRADATION","INTERVENTION","FAILURE"]},

"metadata": {"type":"object"}

}

}

### 4.2 Telemetry Summary — telemetry_summary.json

{

"asset_id":"A123",

"window": {"start":"...","end":"..."},

"signals":[

{"sensor_id":"S1","stats":{"mean":12.3,"std":1.2,"min":10.1,"max":15.8,"pct_change_vs_baseline":36.5},

"anomalies":[{"timestamp":"...","type":"changepoint","score":0.8}],

"changepoints":[{"timestamp":"...","method":"CUSUM"}]

}

]

}

### 4.3 KG Context — kg_context.json

{

"event_id":"E123",

"components":[{"component_id":"C1","role":"valve","model":"X12","sensors":["S1","S2"]}],

"upstream_paths":[{"from":"C0","to":"C1","path":["C0","C2","C1"], "path_strength":0.72}],

"failure_modes":[{"fm_id":"FM_therm_overheat","component_id":"C1","detection":"temp_sensor","expected_latency":{"min_hours":0,"max_hours":24}}]

}

### 4.4 Causality Candidate Edge — causality_candidates.json (list)

{

"source_event":"E_old",

"target_event":"E_now",

"kg_path":["E_old","C2","C1","E_now"],

"score":0.74,

"components_involved":["C1","C2"],

"confidence":{"structural":0.8,"evidence":0.6,"temporal":0.9,"governance":0.4,"overall":"MEDIUM"}

}

### 4.5 Ishikawa Matrix — ishikawa_matrix.json

{

"event_id":"E_now",

"branches":{

"People":{"findings":["crew_shift_change"],"evidence":["doc_123#L10-L20"],"confidence":0.6},

"Methods": {...},

"Machines": {...},

"Materials": {...},

"Measurements": {...},

"Environment": {...},

"Management": {...}

}

}

### 4.6 RCA Card — rca_card.json

{

"event_id":"E_now",

"primary_hypothesis":{"fm_id":"FM_x","description":"Loss of lubrication → bearing wear → vibration trip","confidence": { ... }},

"alternatives":[ ... ],

"evidence":[{"doc_id":"doc_123","section":"3.1","snippet":"...","rank":1}],

"recommended_actions":[{"action_id":"A1","description":"Inspect bearing and replace lubricant","impact_est":"HIGH","effort_est":"LOW","owner":"maintenance"}],

"provenance":{"kg_paths":[...],"telemetry_summary":"telemetry_summary.json","created_by":"LLM_v1.1","created_at":"..."}

}

(Place full, machine-valid JSON Schema files in spec/schemas/ in the repo.)

## 5. Knowledge Graph (Neo4j) Design

### 5.1 Node Types

- :Event {event_id, type, timestamp_start, timestamp_end, severity, asset_id, description}

- :Component {component_id, model_id, location_id}

- :Sensor {sensor_id, type, calibration_date}

- :FailureMode {fm_id, name, superclasses, mechanism}

- :Procedure {procedure_id, revision, text_hash}

- :PMPlan {plan_id, schedule, asset_id}

- :ECR {ecr_id, status, implemented_date}

- :RCA_Card {rca_id, event_id, summary}

### 5.2 Edge Types & Properties

- (:Component)-[:HAS_SENSOR]->(:Sensor)

- (:Component)-[:UPSTREAM_OF {path_strength}] ->(:Component) (directed)

- (:Event)-[:OCCURRED_ON]->(:Component)

- (:FailureMode)-[:APPLIES_TO]->(:Component)

- (:Event)-[:CONFIRMED_CAUSE {confidence, evidence_refs}]->(:FailureMode)

- (:Event)-[:MAY_CAUSE {confidence, lag_min, lag_max, provenance}]->(:Event)

- (:Procedure)-[:GOVERNS]->(:Component)

### 5.3 Edge Weighting & Path Strength

- Path strength for an N-hop path = product of normalized edge weights * decay_factor^(N-1)

- Edge weights: defined per edge type (e.g., UPSTREAM_OF = 0.9, APPLIES_TO = 0.95, GOVERNS = 0.7). Keep these as parameters in config/kg_weights.yaml.

## 6. Confidence Model (Formal)

Confidence must be multidimensional and rule-based; do not rely on LLM scalar outputs alone.

### 6.1 Dimensions

- structural: completeness and strength of KG path (0..1)

- evidence: number & quality of supporting docs (0..1)

- temporal: temporal plausibility given event times and FM latencies (0..1)

- governance: PM/LCM/ECR compliance & corrective action status (0..1)

- overall: discrete label derived from rule-set {VERY_LOW, LOW, MEDIUM, HIGH, VERY_HIGH}

### 6.2 Aggregation Rules (example)

if structural < 0.2 then overall = VERY_LOW

else if evidence < 0.2 and structural < 0.5 then overall = LOW

else overall_score = 0.4*structural + 0.3*evidence + 0.2*temporal + 0.1*governance

map overall_score: [0-0.2]=VERY_LOW, (0.2-0.4]=LOW, (0.4-0.65]=MEDIUM, (0.65-0.85]=HIGH, >0.85=VERY_HIGH

Keep rules in a param file config/confidence_rules.yaml and expose toggles for sensitivity (conservative vs permissive).

## 7. Temporal Reasoning — Formalism

### 7.1 Event Time Windows & Uncertainty

Each event has [t_start, t_end] and associated uncertainty σ_start, σ_end. FMs have expected propagation latencies [min, nominal, max].

A causal relation E_h → E_now is temporally plausible if:
E_h.timestamp_end + FM.min - σ <= E_now.timestamp_start + σ AND E_h.timestamp_end + FM.max + σ >= E_now.timestamp_start - σ
(where σ is combined uncertainty).

Score temporal plausibility as:

temporal_score = max(0, 1 - (distance_from_expected / expected_range))

Distance_from_expected = how far E_now falls outside the expected latency interval (normalized).

### 7.2 Concurrent & Seasonal Patterns

Compute recurrence & seasonal correlation using SARIMA or a simpler frequency test; produce frequency_score used as a feature for causality ranking.

## 8. Algorithms & Pseudocode

### 8.1 Step 1 — Event Framing (deterministic)

- Input: event record

- Action: validate event fields, set analysis window t_event ± Δt (Δt default 24–72 hrs but configurable)

- Output: framed event record

### 8.2 Step 2 — Telemetry Snapshot (deterministic with analytics)

Pseudocode:

function build_telemetry_snapshot(event, sensors, config):

window = [event.t_start - config.pre_window, event.t_end + config.post_window]

for sensor in sensors:

timeseries = telemetry_api.get(sensor, window)

stats = compute_stats(timeseries)

anomalies = detect_anomalies(timeseries)       # z-score, rule-based, changepoint (CUSUM)

changepoints = detect_changepoints(timeseries) # CUSUM or offline algorithm

store per-sensor summary

return telemetry_summary

### 8.3 Step 3 — KG Narrowing (graph query + filter)

Pseudocode:

function kg_narrow(event):

components = kg.query("MATCH (c:Component)-[:HAS_ASSET]->(a) WHERE a.asset_id = $asset RETURN c")

neighbors = expand_components(components, depth=config.depth)

fetch_failure_modes(neighbors)

fetch_past_events_linked(neighbors, within=config.past_event_window)

return kg_context (components, paths, FMs, past_events, SOPs, FMEA)

- implement expand_components with parameters for depth and path_strength threshold.

### 8.4 Step 4 — Causality Candidate Generation & Scoring

This is the core algorithm.

Pseudocode:

function propose_causality(event_now, kg_context, telemetry_summary):

candidates = []

for E_h in kg_context.past_events:

if not temporal_precedence(E_h, event_now): continue

paths = kg.find_paths(E_h, event_now, max_hops=config.max_hops)

for path in paths:

structural = compute_path_strength(path)

evidence = evidence_overlap_score(path, kg_context, telemetry_summary)

temporal = temporal_plausibility(E_h, event_now, path)

governance = governance_overlap_score(E_h, event_now)

combined_score = weighted_sum([structural,evidence,temporal,governance], weights=config.weights)

if combined_score >= config.candidate_threshold:

candidates.append({E_h, path, combined_score, confidence:{...}})

rank candidates by combined_score

return causality_candidates

Notes:

- compute_path_strength uses edge weights and decay.

- evidence_overlap_score looks for matching FM names, SOP references, and telemetry signatures.

- Use early pruning: if path contains edges with path_strength < min_edge, discard.

### 8.5 Step 5 — Evidence Retrieval

- Construct vector search queries from KG context: "{component_name} {fm_name} {procedure_id} {asset_model}".

- Use result ranking by similarity and metadata match (asset_id, location, procedure_id).

- Produce evidence_bundle.json with score, doc_id, section, snippet, image_refs.

### 8.6 Step 6 — Ishikawa Evaluation (rule + scoring)

- For each branch, enumerate checks (from doc: People, Methods, Machines, Materials, Measurements, Environment, Management).

- For each check produce:

  - deviation_flag (True/False)

  - evidence_refs (doc ids/snippets)

  - branch_confidence (0..1)

- Interaction logic: compute pairwise interaction weights; e.g., Methods * People increases overall causal weight.

Pseudocode:

for branch in branches:

findings = run_checks(branch, evidence_bundle, ops_context, telemetry)

branch_confidence = combine_findings(findings)

ishikawa_score = aggregate_branch_scores(branch_confidences, interaction_matrix)

### 8.7 Step 7 — Effectiveness & Compliance

- Query PM execution records for asset within window; calculate PM_compliance = executed/planned.

- Evaluate prior corrective actions by measuring pre/post metrics (anomaly rate, vibration RMS).

- Output structured compliance summary.

### 8.8 Step 8 — LLM Synthesis (constrained)

LLM prompt template includes:

- short "context" (event, telemetry summary, top 5 KG paths with short path strings)

- enumerated evidence items (top 10)

- Ishikawa matrix (branches with findings)

- request to produce: primary hypothesis, alternatives, recommended actions, confidence mapping, unknowns, explicit KG path citations.

Important constraints:

- LLM must produce JSON matching rca_card.json schema.

- The pipeline must validate JSON and that each claim has at least one evidence_ref unless labeled as SPECULATIVE (and given low confidence).

### 8.9 Step 9 & 10 — Analyst Review & Write-back

- Present rca_card and raw evidence to analyst UI.

- Analyst can set status = {ACCEPTED, MODIFIED, REJECTED} and add sign-off.

- On ACCEPTED: commit edges to KG per schema and store RCA in Chroma (text + structured fields).

Commit pseudo-operation:

kg.commit("MERGE (e:Event{event_id:...}) ...

CREATE (e)-[:CONFIRMED_CAUSE{confidence:...}]->(fm)

CREATE (e)-[:MITIGATED_BY]->(action)

## 9. Implementation Notes & Developer Guidance (for DACKAR repo)

- Provide config files: config/kg_weights.yaml, config/confidence_rules.yaml, config/telemetry_windows.yaml.

- Split repo into modules:

  - ingest/ (telemetry, ops, cmms connectors)

  - kg/ (neo4j client, queries, commit helpers)

  - causal/ (causality engine, scoring)

  - evidence/ (chroma client, retrieval)

  - ishikawa/ (branch checks)

  - synthesis/ (LLM prompt templates & validation)

  - ui/ (review UI stub)

  - spec/ (schemas + method doc)

  - tests/ (unit & integration)

- Use TypeScript or Python for +/- type-safety. (If INL prefers Python for data science stacks, use Python + FastAPI for endpoints.)

## 10. Validation, Tests & Acceptance Criteria

### 10.1 Unit Tests (examples)

- test_telemetry_summary: given synthetic timeseries with known changepoints, assert detection.

- test_kg_path_strength: given a toy KG, assert path_strength values match expected.

- test_temporal_plausibility: edge cases for latency windows.

- test_confidence_rules: rules map to expected overall label for crafted inputs.

- test_llm_output_schema: LLM output validator rejects missing evidence_refs.

### 10.2 Integration Tests

- Synthetic scenario ingest (create fake events, telemetry, past events) → full pipeline → outputs rca_card with expected primary hypothesis.

- End-to-end test including KG commit for accepted RCA, and subsequent retrieval for a follow-up event to ensure recurrence metrics update.

### 10.3 Acceptance Criteria

- Reproducibility: For the same inputs, pipeline produces identical causality_candidates.json.

- Grounding: Each primary causal claim must reference ≥1 evidence item (doc_id+section) or be explicitly marked SPECULATIVE.

- Audit trail: All KG commits carry author, timestamp, and provenance.

- Explainability: rca_card.md must include a KG path diagram or list and an Ishikawa matrix.

## 11. Security, Privacy & Regulatory Considerations

- Sanitize evidence extracts to avoid PII or classified text leaking into models if necessary.

- Maintain immutable audit logs for all KG commits and analyst sign-offs (for NRC/INPO audits).

- LLM usage: keep prompts and retrieved evidence internal to environment (no external model without approval).

## 12. Example Walkthrough (compact)

Provide an example JSON flow for a sample event (sketch only — full example files belong in spec/examples/).

- Event E_now (id E2026-01-01-001) triggers.

- Telemetry summary shows sudden temperature increase on sensor S1.

- KG narrowing finds Component C_pump_1, FM FM_loss_lubrication, past event E_2025-09-17 with confirmed bearing wear.

- Causality engine proposes E_2025-09-17 -> E_now with path through shared maintenance crew and same lubricant batch, combined_score=0.78.

- Evidence retriever returns WO docs showing lubricant change with different spec; Ishikawa highlights Materials and Methods.

- LLM synthesizes hypothesis: wrong lubricant spec → premature bearing wear → increased temperature → protection trip. Provides actions: inspect bearing, verify lubricant spec across site.

- Analyst accepts; KG updated: (E_now)-[:CONFIRMED_CAUSE]->(FM_loss_lubrication) and (E_2025-09-17)-[:MAY_CAUSE]->(E_now).

## 13. Deliverables for the Repo (what to add now)

- spec/method_spec.md — this document (canonical)

- spec/schemas/*.json — JSON schemas above for all outputs

- spec/config/*.yaml — default weights and rules

- spec/examples/ — at least 2 worked examples (one simple synthetic, one anonymized historical)

- src/ scaffolding per modules listed in Section 9

- tests/ — unit & integration test stubs

- README.md — quickstart for developers and system engineers

Note on dackar_spec_demo.zip

dackar_spec_demo/

├── README.md

├── spec/

│   ├── schemas/          # Formal JSON Schemas (method specification)

│   └── examples/         # Synthetic example event

├── src/

│   ├── ingest/           # Telemetry ingestion skeleton

│   ├── kg/               # Knowledge graph client skeleton

│   ├── causal/           # Causality engine skeleton

│   ├── evidence/         # Evidence retrieval (vector store) skeleton

│   ├── ishikawa/         # Ishikawa evaluator skeleton

│   └── synthesis/        # LLM synthesis & validation skeleton

├── data/                 # Synthetic telemetry data

└── tests/                # Pytest unit test stubs

# Notation

Equipment: A physical, uniquely identified instance in the plant

Examples:

Pump P-101A

Diesel Generator DG-2

Valve V-34-B

Properties:

asset_id (unique)

location (room/building)

operational history

telemetry streams

PM execution records

CRs, WOs, alarms, trips

Component: Engineering element defined by the system architecture (MBSE).

Examples:

Rolling-element bearing

Shaft seal

Motor winding

Properties:

component_id (design-level)

failure modes (FMEA)

upstream/downstream dependencies

SOP references

manufacturer design specs

Components are where failure modes live.

Assets are where events occur.