## 1. Telemetry Historian API

All endpoints are read-only. Base path: /api/v1/telemetry. Timestamps follow ISO 8601 / RFC 3339. All responses are JSON.

The split between /timeseries and /timeseries/batch is intentional — the single-sensor call is used when matrix profile is run on individual signals, while the batch call is used at Step 2 startup to retrieve all sensors for an asset without N serial round trips. The /timeseries/baseline endpoint has three strategies (fixed_window, rolling_days, last_stable_period) because the right baseline depends on context — a rolling window is appropriate for routine analysis but the last KG-confirmed stable period is more defensible for a formal RCA. The /events/history endpoint is what feeds the TSKR mining step — it returns the historical validated anomaly event corpus scoped to an asset, typed and MMD-validated, which is exactly what Step 3A consumes.

### 1.1 GET /sensors

List available sensors for an asset.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| asset_id | string | ✅ | Asset identifier |
| parameter_type | string | ❌ | Filter by parameter (e.g., temperature, vibration) |

Response:

json

{

"asset_id": "P-101A",

"sensors": [

{

"sensor_id": "S-T-101",

"parameter": "bearing_temp",

"unit": "degC",

"component_id": "C-BEARING-101A",

"sampling_interval_seconds": 300,

"resolution_seconds": 300,

"native_temporal_type": "interval_native",

"engineering_limits": {

"low_low": 0.0, "low": 20.0,

"high": 75.0, "high_high": 90.0,

"unit": "degC"

},

"calibration_date": "2025-06-01T00:00:00Z",

"calibration_due":  "2026-06-01T00:00:00Z",

"active": true

}

]

}

### 1.2 GET /timeseries

Fetch raw univariate time series for a sensor over a window. Primary input to matrix profile.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| sensor_id | string | ✅ | Sensor identifier |
| asset_id | string | ✅ | Asset identifier |
| from | datetime | ✅ | Window start (inclusive) |
| to | datetime | ✅ | Window end (inclusive) |
| resample_interval_seconds | integer | ❌ | Resample to uniform interval. Default: native sampling interval |
| fill_method | string | ❌ | Gap-fill method: forward_fill, linear, none. Default: forward_fill |
| quality_filter | boolean | ❌ | Exclude flagged bad-quality samples. Default: true |

Response:

json

{

"sensor_id": "S-T-101",

"asset_id":  "P-101A",

"parameter": "bearing_temp",

"unit": "degC",

"from": "2025-12-29T03:14:00Z",

"to":   "2026-01-03T03:45:00Z",

"resample_interval_seconds": 300,

"fill_method": "forward_fill",

"sample_count": 8640,

"missing_count": 9,

"missing_fraction": 0.001,

"values": [

{"timestamp": "2025-12-29T03:14:00Z", "value": 63.2, "quality": "good"},

{"timestamp": "2025-12-29T03:19:00Z", "value": 63.4, "quality": "good"}

]

}

### 1.3 GET /timeseries/batch

Fetch multiple sensors for an asset in a single call. Used at Step 2 to retrieve all sensors associated with the event asset without N serial calls.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| asset_id | string | ✅ | Asset identifier |
| from | datetime | ✅ | Window start |
| to | datetime | ✅ | Window end |
| sensor_ids | string (comma-separated) | ❌ | Subset of sensors. Default: all active sensors for asset |
| resample_interval_seconds | integer | ❌ | Applied uniformly to all signals |
| fill_method | string | ❌ | Applied uniformly |

Response:

json

{

"asset_id": "P-101A",

"from": "2025-12-29T03:14:00Z",

"to":   "2026-01-03T03:45:00Z",

"signals": [

{

"sensor_id": "S-T-101",

"parameter": "bearing_temp",

"unit": "degC",

"sample_count": 8640,

"missing_fraction": 0.001,

"values": [ ... ]

},

{

"sensor_id": "S-V-101",

"parameter": "shaft_vibration_x",

"unit": "mm/s",

"sample_count": 8640,

"missing_fraction": 0.0,

"values": [ ... ]

}

]

}

### 1.4 GET /timeseries/baseline

Fetch the baseline window for a sensor. Called separately from /timeseries so that the baseline period is independently configurable and cacheable.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| sensor_id | string | ✅ | Sensor identifier |
| asset_id | string | ✅ | Asset identifier |
| baseline_strategy | string | ❌ | fixed_window (use from/to), rolling_days (N days before event), last_stable_period (last KG-confirmed stable period). Default: fixed_window |
| from | datetime | ❌ | Required if baseline_strategy=fixed_window |
| to | datetime | ❌ | Required if baseline_strategy=fixed_window |
| rolling_days | integer | ❌ | Required if baseline_strategy=rolling_days |
| event_id | string | ❌ | Required if baseline_strategy=last_stable_period — uses KG to identify last period without confirmed anomaly |
| resample_interval_seconds | integer | ❌ | Must match the analysis window resample interval for valid comparison |

Response: Same structure as /timeseries.

### 1.5 GET /engineering-limits

Fetch defined engineering and alarm limits for a sensor. Used by matrix profile promotion gate to check limit exceedance.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| sensor_id | string | ✅ | Sensor identifier |
| asset_id | string | ✅ | Asset identifier |

Response:

json

{

"sensor_id": "S-T-101",

"asset_id":  "P-101A",

"parameter": "bearing_temp",

"unit": "degC",

"limits": {

"low_low":   {"value": 0.0,  "action": "trip"},

"low":       {"value": 20.0, "action": "alarm"},

"high":      {"value": 75.0, "action": "alarm"},

"high_high": {"value": 90.0, "action": "trip"}

},

"source_ref": "SOP-LUBE-012 §2.1",

"effective_date": "2024-01-15T00:00:00Z"

}

### 1.6 GET /events/history

Fetch historical anomaly event records for an asset within a lookback window. Used by Step 3A (TSKR mining) to build the historical event corpus for pattern mining.

Query parameters:

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| asset_id | string | ✅ | Asset identifier |
| from | datetime | ✅ | Lookback start |
| to | datetime | ✅ | Lookback end |
| event_types | string (comma-separated) | ❌ | Filter by event type. Default: all |
| validated_only | boolean | ❌ | Return only MMD-validated events. Default: true |
| include_sensor_ids | boolean | ❌ | Include originating sensor list. Default: false |

Response:

json

{

"asset_id": "P-101A",

"from": "2023-01-01T00:00:00Z",

"to":   "2026-01-01T00:00:00Z",

"events": [

{

"event_id":        "KGE-2025-0047",

"event_type":      "BEARING_TEMP_ANOMALY",

"temporal_type":   "interval_native",

"t_in":            "2025-09-15T10:00:00Z",

"t_end":           "2025-09-17T14:00:00Z",

"severity_score":  0.79,

"mmd_validated":   true,

"sensor_ids":      ["S-T-101"],

"fm_id":           "FM-BEAR-WEAR-001",

"source":          "matrix_profile"

}

]

}

### 1.7 Error Schema (all endpoints)

json

{

"error_code": "SENSOR_NOT_FOUND",

"message":    "Sensor S-T-999 not found for asset P-101A.",

"request_id": "req-abc123",

"timestamp":  "2026-01-01T04:00:00Z"

}

Standard error codes:

| Code | HTTP Status | Meaning |
| --- | --- | --- |
| SENSOR_NOT_FOUND | 404 | Sensor/asset combination does not exist |
| WINDOW_TOO_LARGE | 400 | Requested window exceeds config.max_window_days |
| NO_DATA_IN_WINDOW | 204 | Valid sensor, no data in requested window |
| INSUFFICIENT_QUALITY | 206 | Data returned but missing fraction exceeds threshold |
| HISTORIAN_UNAVAILABLE | 503 | Backend historian unreachable |